TVD-M3245E-2M-N(-P) IP Dome Camera A&E Specifications, Division 28 00 00 Electronic Safety and Security

P/N 1072852-EN • REV B • ISS 24FEB17

This A&E Specification conforms to CSI MasterFormat 2016 guidelines.

28 05 00 Common Work Results for Electronic Safety and Security

28 05 07 Power Sources for Electronic Safety and Security

28 05 07.21 Poe Power Sources for Electronic Safety and Security

# TVD-M3245E-2M-N(-P) shall support High PoE (802.3at) power supply.

28 05 21 Network Attached Storage for Electronic Safety and Security

# TVD-M3245E-2M-N(-P) shall record video on NAS.

28 05 45 Systems Integration and Interconnection Requirements

# Connections

## TVD-M3245E-2M-N(-P) shall include the following connectors:

### 1, RJ45 10 M / 100 M adaptive Ethernet port

### Alarm input and output

# Ethernet communications

## TVD-M3245E-2M-N(-P) shall support LAN/WAN Ethernet access.

## TVD-M3245E-2M-N(-P) shall support 10/100 Base T networks.

## TVD-M3245E-2M-N(-P) shall support Dynamic IP Addressing (DHCP).

## TVD-M3245E-2M-N(-P) shall support Dynamic Domain Name Server (DDNS).

28 05 45.11 Mechanical

# Mechanical

## Dimensions: Ø 160 × 146 mm (Ø 6.29 × 5.75 in.)

## Weight: 2100 g (4.62 lb.)

28 05 45.13 Electrical

# Electrical

## Power supply: 24 VAC ±10%, PoE (IEEE 802.3af – heater will be disabled), High PoE (802.3at)

28 05 45.15 Information

# Environmental

## Operating temperature range: -40 to +60°C (-40 to +140°F)

## Enclosure ratings:

### Impact Protection: IEC60068-2-75Eh, 50J; EN50102, up to IK10

### IP66

# Compliance

## FCC

## CE

## UL

## C-Tick

28 05 53 Identification for Electronic Safety and Security

# The TVD-M3245E-2M-N(-P) IP 3MPX WDR Dome camera shall capture, encode, and transmit video over a network.

# The TVD-M3245E-2M-N(-P) shall be as manufactured by Interlogix.

28 20 00 Video Surveillance

28 21 00 Surveillance Cameras

28 21 13 IP Cameras

# The TVD-M3245E-2M-N(-P) shall support the encoding of all images with a digital watermark. The verification of watermarked images shall reside solely with Interlogix.

# TVD-M3245E-2M-N(-P) shall include, but not be limited to the following:

## TVD-M3245E-2M-N(-P) shall provide network connections for the purpose of allowing users to integrate it with network storage products.

### TVD-M3245E-2M-N(-P) shall provide 1/3” Progressive Scan CMOS sensor.

### TVD-M3245E-2M-N(-P) shall provide digital encoded video stream.

## TVD-M3245E-2M-N(-P) shall support High PoE power supply.

## TVD-M3245E-2M-N(-P) shall provide IR Cut filter.

## TVD-M3245E-2M-N(-P) shall provide infrared LED.

## TVD-M3245E-2M-N(-P) shall provide WDR.

## TVD-M3245E-2M-N(-P) shall provide RCA output。

## TVD-M3245E-2M-N(-P) shall provide 2.7 – 9 mm @ F1.6 motorized lens with zoom function.

## TVD-M3245E-2M-N(-P) shall provide a 3D noise reduction function.

## TVD-M3245E-2M-N(-P) shall provide a Micro SD card for on-board storage.

## TVD-M3245E-2M-N(-P) shall provide an on-board reset button.

## TVD-M3245E-2M-N(-P) shall provide dual streams.

## TVD-M3245E-2M-N(-P) shall incorporate Triplex functionality for simultaneous viewing, playback and recording (By web browser).

## TVD-M3245E-2M-N(-P) shall include search capabilities by a web browser:

### Time

### Date

## TVD-M3245E-2M-N(-P) shall provide a frame rate of 30 frames per second @ 60Hz (25 frames per second @ 50 Hz) at the resolution of 1920×1080.

## TVD-M3245E-2M-N(-P) shall be user configured via Ethernet with a personal computer running included, and a current version of the Internet Explorer web browser.

### TVD-M3245E-2M-N(-P) shall have an integrated web client interface to configure, upgrade, and view the following information:

#### View live and recorded video

#### View logs of system

#### Configure system settings, which include network settings, and network HDD settings

#### Configure camera settings and user settings

#### View the system information

### Access to menus shall be set by user status.

#### Operator status shall grant access to change the configuration of his/her own account, and cannot create or delete other users.

#### Viewer status shall grant access to live view, record and playback modes as well as take snapshots, but neither change configuration settings nor search the logs for events.

#### Admin status shall grant access to all menus.

### Configurable options shall include:

#### Search

##### Date

##### Time

#### Archive

#### Display

##### Time/Date

##### Playback Time/Date

##### Camera Titles

##### Text Insertion

#### Users

##### Add User

##### Edit User

#### Cameras

##### Resolution, frame rate and bit rate

##### Motion detection

##### Camera Title and Date/Time

##### Record schedule

#### Image

##### Brightness

##### Contrast

##### Video saturation

##### Hue

##### Sharpness

##### Shutter Speed

##### Iris Mode

##### Power Mode

##### IR Mode

##### Day/Night

##### IR Cut Delay

##### Image flip

##### WDR

##### White balance

##### SceneMode

#### Alarms

##### System Notification

###### HDD Error

###### HDD Full

###### Network Disconnected

###### IP Address conflicted

###### Illegal Login

#### IP Settings

##### Basic Settings

##### PPPoE

##### DDNS

##### Email

##### Net HDD

# The IP bullet camera shall have the following operational features:

## Streaming

### Each TVD-M3245E-2M-N(-P) shall support Video Streaming, which is the process that the recorder uses to listen on a specific UDP/TCP port and respond to control messages issued through web client software or third-party compatible VMS software.

#### TVD-M3245E-2M-N(-P) shall support multicasting to deliver source traffic to multiple receivers using the least amount of network bandwidth.

### TVD-M3245E-2M-N(-P) shall be able to display and record streamed video using TCP or UDP protocols.

### TVD-M3245E-2M-N(-P) shall support PSIA and ONVIF protocols.

## Recording

### TVD-M3245E-2M-N(-P) shall record video on multiple network hard drives.

### TVD-M3245E-2M-N(-P) shall support the following user programmable record speeds:

#### 30/25 fps

#### 22 fps

#### 20 fps

#### 18 fps

#### 16 fps

#### 15 fps

#### 12 fps

#### 10 fps

#### 8 fps

#### 6 fps

#### 4 fps

#### 2 fps

#### 1 fps

#### 1/2 fps

#### 1/4 fps

#### 1/8 fps

#### 1/16 fps

### TVD-M3245E-2M-N(-P) shall support the following bit rate:
32 kbps to 16 Mbps, or user-defined

### TVD-M3245E-2M-N(-P) shall allow the user to select whether the network hard drive recording should automatically overwrite data and how using one of two settings:

#### No overwrite

#### Continuous overwrite

### TVD-M3245E-2M-N(-P) shall be able to continue recording without disruption when the user adjusts the normal record speed.

### The user shall be able to play back videos smoothly at normal or fast speeds and in forward mode, without distortion.

### TVD-M3245E-2M-N(-P) shall include a Search Interface feature that allows the user to search the network hard drive for recorded videos.

### TVD-M3245E-2M-N(-P) shall use H.264 video compression to achieve extremely high video compression per megabyte on the hard drive.

#### Image quality shall be user-selectable when the bit rate type is variable, on a scale of 1 through 6.

#### Resolution shall be 2048×1536, 1920×1080, 1280×960, 1280×720, and 640×480.

Contacting Support

North America:

855-286-8889

techsupport@interlogix.com

Latin America:

561-998-6114

latam@interlogix.com

Web site:

www.interlogix.com/customer-support

EMEA:

See specific country listings at:

www.utcfssecurityproducts.eu/support