TruVision 30X 2MPX IR PTZ Camera A&E Specifications, Division 28 00 00 Electronic Safety and Security

P/N 1073209-EN • REV B • ISS 24FEB17

This A&E Specification conforms to CSI MasterFormat 2016 guidelines.

28 05 00 Common Work Results for Electronic Safety and Security

28 05 07 Power Sources for Electronic Safety and Security

28 05 07.21 Poe Power Sources for Electronic Safety and Security

# The TruVision 30X 2MPX IR PTZ shall support High PoE (802.3at) power supply.

28 05 21 Network Attached Storage for Electronic Safety and Security

# The TruVision 30X 2MPX IR PTZ shall record video on multiple hard drives. Two hard drive options shall be available: NAS and SD card.

28 05 45 Systems Integration and Interconnection Requirements

# Connections

## The TruVision 30X 2MPX IR PTZ shall include the following connectors:

### One RJ45 10 M / 100 M adaptive Ethernet port

### One 1.0V[p-p] / 75Ω, NTSC/PAL composite, BNC

### One audio input; one output

### Seven alarm inputs; two alarm outputs

# Ethernet communications

## The TruVision 30X 2MPX IR PTZ shall support LAN/WAN Ethernet access.

## The TruVision 30X 2MPX IR PTZ shall support 10/100 Base T networks.

## The TruVision 30X 2MPX IR PTZ shall support Dynamic IP Addressing (DHCP).

## The TruVision 30X 2MPX IR PTZ shall support Dynamic Domain Name Server (DDNS).

28 05 45.11 Mechanical

# Mechanical

## Dimensions: Ø 245 × 371 mm

## Weight: 6 kg

28 05 45.13 Electrical

# Electrical

## Power supply: 24 VAC, High PoE (802.3at)

28 05 45.15 Information

# Environmental

## Operating temperature:

### High PoE -0 ~ +65°C

### 24 VAC: -40°C ~ +65°C

## Enclosure ratings:

### IP66

# Compliance

## FCC

## CE

## UL

## C-Tick

## REACH

## RoHS

## WEEE

28 05 53 Identification for Electronic Safety and Security

# The The TruVision 30X 2MPX IR PTZ camera shall capture, encode, and transmit video over a network.

# The The TruVision 30X 2MPX IR PTZ shall be as manufactured by Interlogix.

28 20 00 Video Surveillance

28 21 00 Surveillance Cameras

28 21 13 IP Cameras

# The The TruVision 30X 2MPX IR PTZ shall support the encoding of all images with a digital watermark. The verification of watermarked images shall reside solely with Interlogix.

# The TruVision 30X 2MPX IR PTZ shall include, but not be limited to the following:

## The TruVision 30X 2MPX IR PTZ shall provide network connections for the purpose of allowing users to integrate it with network storage products.

### The TruVision 30X 2MPX IR PTZ camera shall provide 1/2.8” Progressive Scan CMOS sensor.

### The TruVision 30X 2MPX IR PTZ camera shall provide digital encoded video stream.

## The TruVision 30X 2MPX IR PTZ camera shall support High PoE power supply.

## The TruVision 30X 2MPX IR PTZ camera shall provide IR Cut filter.

## The TruVision 30X 2MPX IR PTZ camera shall provide 4.3 to 129 mm @ F1.6 -5.0 lens.

## The TruVision 30X 2MPX IR PTZ camera shall provide noise reduction function.

## The TruVision 30X 2MPX IR PTZ camera shall provide DWDR function.

## The TruVision 30X 2MPX IR PTZ camera shall provide three streams.

## The TruVision 30X 2MPX IR PTZ camera shall incorporate Triplex functionality for simultaneous viewing, playback and recording (By web browser).

## The TruVision 30X 2MPX IR PTZ camera shall include search capabilities by web browser:

### Time

### Date

## The TruVision 30X 2MPX IR PTZ camera shall provide a frame rate of 30 frames per second @ 60 Hz (25 frames per second @ 50 Hz) at the resolution of 1920×1080.

## The TruVision 30X 2MPX IR PTZ camera shall be user configured via Ethernet with a personal computer running a current version of the Internet Explorer web browser.

### The TruVision 30X 2MPX IR PTZ camera shall have an integrated web client interface to configure, upgrade, and view the following information:

#### View live and recorded video

#### View logs of system

#### Configure system settings, which include network settings, and network HDD settings

#### Configure camera settings and user settings

#### View the system information

### Access to menus shall be set by user status.

#### Operator status shall grant access to change the configuration of his/her own account, and cannot create or delete other users.

#### Viewer status shall grant access to live view, playback modes as well as log search.

#### Admin status shall grant access to all menus.

### Configurable options shall include:

#### Search

##### Date

##### Time

#### Archive

#### Display

##### Time/Date

##### Playback Time/Date

##### Camera Titles

##### Text Insertion

#### Users

##### Add User

##### Edit User

#### Cameras

##### Resolution, frame rate and bit rate

##### Motion detection

##### Camera Title and Date/Time

##### Record schedule

#### Image

##### Brightness

##### Contrast

##### Saturation

##### Sharpness

##### Exposure Mode

##### Limit Gain

##### Sharpness

##### Focus Mode

##### Minimum Focus Distance

##### Day/Night Switch

##### Sensitivity

##### BLC

##### DWDR

##### White Balance

##### Digital Noise Reduction

##### Mirror

##### Video Standard

##### Capture Mode

##### Lens Initialization

##### Zoom Limit

##### Local Output

#### Alarms

##### System Notification

###### HDD Error

###### HDD Full

###### Network Disconnected

###### IP Address conflicted

###### Illegal Login

#### PTZ Settings

##### Basic settings

##### Limit

##### Initial Position

##### Park Action

##### Privacy Mask

##### Scheduled Tasks

##### Clear Config

##### Intelligent Tracking

#### IP Settings

##### TCP/IP

##### Port

##### DDNS

##### PPPoE

##### SNMP

##### 802.1X

##### QoS

##### FTP

##### UPnP

##### Email

##### NAT

##### Net HDD

# The TruVision 30X 2MPX IR PTZ camera shall have the following operational features:

## Streaming

### Each TruVision 30X 2MPX IR PTZ camera shall support Video Streaming, which is the process that the recorder uses to listen on a specific UDP/TCP port and respond to control messages issued through web client software or third-party compatible VMS software.

#### The TruVision 30X 2MPX IR PTZ shall support multicasting to deliver source traffic to multiple receivers using the least amount of network bandwidth.

### The TruVision 30X 2MPX IR PTZ shall be able to display and record streamed video using TCP or UDP protocols.

### The TruVision 30X 2MPX IR PTZ shall support PSIA and ONVIF protocols.

## Recording

### The TruVision 30X 2MPX IR PTZ shall record video on multiple hard drives. Two hard drive options shall be available: NAS and SD card.

### The TruVision 30X 2MPX IR PTZ shall support the following user programmable record speeds:

#### 30/25 fps

#### 22 fps

#### 20 fps

#### 18 fps

#### 16 fps

#### 15 fps

#### 12 fps

#### 10 fps

#### 8 fps

#### 6 fps

#### 4 fps

#### 2 fps

#### 1 fps

#### 1/2 fps

#### 1/4 fps

#### 1/8 fps

#### 1/16 fps

### The TruVision 30X 2MPX IR PTZ shall support the following bit rate: 128 kbps to 16 Mbps, or user-defined.

### The TruVision 30X 2MPX IR PTZ shall allow the user to select whether the network hard disk recording should automatically overwrite data and how using one of two settings:

#### No overwrite

#### Continuous overwrite

### The TruVision 30X 2MPX IR PTZ shall be able to continue recording without disruption when the user adjusts the normal record speed.

### The user shall be able to play back videos smoothly at normal or fast speeds and in forward mode, without distortion.

### The TruVision 30X 2MPX IR PTZ shall include a Search Interface feature that allows the user to search the network hard drive for recorded videos.

### The TruVision 30X 2MPX IR PTZ shall use H264 video compression to achieve extremely high video compression per megabyte on the hard drive.

#### Image quality shall be user-selectable when the bit rate type is variable, on a scale of 1 through 6.

#### Resolution shall be 1920×1080, 1280×960, 1280×720, 704×576 (704×480), 352×288 (352×240) and 176×144 (176×120).

Contacting Support

North America:

855-286-8889

techsupport@interlogix.com

Latin America:

561-998-6114

latam@interlogix.com

Web site:

www.interlogix.com/customer-support

EMEA:

See specific country listings at:

www.utcfssecurityproducts.eu/support