### TVC-M1120-1-N

### TruVision MegaPixel IP High-Performance Box Camera

- Traditional box housing

  - The TVC-M1120-1-N shall function as a general-purpose video surveillance camera in indoor or outdoor applications.

  - The TVC-M1120-1-N shall be as manufactured by Interlogix or an approved equal.

  - The camera housing shall be constructed of aluminum.

    - Camera shall accept auto-Iris C or CS mounted DC AI lenses.

  - The camera shall meet or exceed the following specifications:

    - The sensor shall be a 1/3-inch format, 1.3 Mega-Pixel CCD image sensor.

      - Effective Pixels (H xV): 1280 × 960, 1.3MPX (12.5 FPS)

      - Minimum FPS at 1280 x 720p resolution to be 24 FPS.

    - Real-time video streaming (30 FPS) at 640 x 480 resolution minimum.

    - The analog, composite video output will be  NTSC standard.

      - The video output shall be 1.0 V p-p at 75-ohm load

    - The composite video output shall be DC internal synchronization.

    - The unit will provide dual IP streams for managing bandwidth usage.

    - The sensitivity shall be 0.1 Lux (color mode) and 0.01 Lux (night mode) @ F1.2.

    - The signal-to-noise ratio shall be better than 50 dB.

    - Exposure/Shutter to be ¼ second to 1/100,000 second.

    - The camera shall be true day night with auto removable IR cut filter

    - The camera shall have Auto color balance and white balance as AWB or ATB.

    - The aperture correction shall be 0 to 15 steps, adjustable.

    - The camera shall offer two-way audio channel.

      - Compression – OggVorbis, 16 kBit/s

      - Input: 1 channel 3.5 mm audio interface (2.0 - 2.4 Vp-p, 1 kOhm)

      - Output: 1 channel 3.5 mm audio interface (Line level, 600 Ohm)

    - Camera shall have a Media SDHC card slot, 16 GB capacity for recording

      - Edge recording capable

    - Camera shall include an integrated browser.

    - Intelligence in the Camera

      - Motion Detection: On-board, 396 zones per camera (22 x 18 grid)

      - Privacy Masking: On-board, 396 zones per camera (22 x 18 grid)

      - Health Check: On-board

18.    Camera shall have Alarm Handling:

      - Alarm Input: 1,  Alarm Output: Form C alarm relay (1)

  - Network:

    - Networkable via Ethernet (TCP/IP) for remote functions

    - Dual Streaming: Main Stream and Sub Stream up to real-time CIF

    - Protocol: TCP/IP, HTTP, DHCP, DNS, RTP/RTCP, PPPoE

    - Type: 1 RJ45 10M / 100M self-adaptive Ethernet port

    - Compression: H.264 or MPEG4

  - The electrical specifications for the dome shall be as follows:

    - The camera shall have a universal input voltage of 12 VDC or 24 VAC, POE.

    - Power consumption shall be 4 watts max.

      - 10 watts max peak when IR filter motor is activated.

  - The environmental specifications for the dome shall be as follows:

    - Operating temperature shall be -10 to 60 degrees Celsius.

  - The physical specifications for the dome shall be as follows:

    - Dimensions shall be 68 x 63 x 157 mm (2.71" x 2.5" x 6.25").

    - Weight shall be 600 g (1.32 lbs).

  - The dome shall conform to these internationally recognized compliance standards:

    - FCC

    - CE

    - UL

    - RoHS