TVC-M3220-1 Series

### TruVision 3 Megapixel, Box, High-Performance Camera

  - The TVC-M3220-1 shall function as a general-purpose video surveillance camera in indoor applications and outdoor applications when used with the appropriate enclosure.

  - The camera shall include a secondary analog video output inside the dome bubble for local test monitor during setup.

  - The camera shall meet or exceed the following specifications:

    - The sensor shall be a Progressive Scan, 1/2.5-inch format (1/3” usable), CMOS image sensor.

      - Maximum Resolution (H x V): 2048 x 1536 (15 FPS)

      - Maximum Real Time Resolution (H x V): 1920 x 1080 (30FPS)

    - Lens:  C/CS mount

    - The camera shall have an NTSC or PAL composite video output (analog format based on model).

      - The video output shall be 1.0 V p-p at 75-ohm load

    - The composite video output shall be DC internal synchronization.

    - The unit will provide dual IP streams for managing bandwidth usage.

      - Main stream shall be a configurable H.264 or MPEG4 stream.

      - Sub stream shall be a configurable H.264, MPEG4 or MJPEG stream.

    - All models shall feature shall feature true day night with auto removable IR cut filter.

    - The sensitivity shall be 0.6 Lux (color and B/W mode).

    - The signal-to-noise ratio shall be better than 50 dB.

    - Exposure/Shutter to be ¼ second to 1/100,000 second.

    - The camera shall have Auto color balance and white balance as AWB.

    - The camera shall offer two-way audio channel.

      - Compression – OggVorbis, 16 kBit/s

      - Input: 1 channel 3.5 mm audio interface (2.0 - 2.4 Vp-p, 1 kOhm)

      - Output: 1 channel 3.5 mm audio interface (Line level, 600 Ohm)

    - Camera shall have a Media SDHC card slot, up to 32 GB capacity for recording

      - Edge Recording capable via schedule, motion detection or alarm

      - Log file recording

    - Intelligence in the Camera

      - Motion Detection: On-board, 396 zones per camera (22 x 18 grid)

      - Privacy Masking: On-board, 396 zones per camera (22 x 18 grid)

    - Camera shall have Alarm Handling:

      - Alarm Input: 1,  Alarm Output: 1 Form C alarm relay

  - Communication Protocols

    - The camera shall support PSIA and ONVIF communication and streaming standards.

    - The camera shall support CGI/HTTP command set.

  - Network:

    - Networkable via Ethernet (TCP/IP) for remote functions

    - Dual Streaming: Configurable Main Stream and Sub Stream

    - Adjustable variable or constant bitrates from 32 Kbps to 8Mbps

    - Protocol: TCP/IP, HTTP, DHCP, DNS, RTP/RTCP, PPPoE

    - Type: 1 RJ45 10M / 100M self-adaptive Ethernet port

    - Compression: H.264/MPEG4 (main) or H.264/MJPEG/MPEG4 (sub stream)

  - The electrical specifications for the dome shall be as follows:

    - The camera shall have an input voltage of 12 VDC or POE (802.3af).

    - Power consumption shall be 5.5 watts max.

      - 10.5 watts max peak with IR filter motor active

  - The environmental specifications for the dome shall be as follows:

    - Operating temperature shall be -10 to 60 degrees Celsius.

  - The physical specifications for the dome shall be as follows:

    - Dimensions shall be 68 x 57x 144mm (2.7 x 2.3 x 5.7 in.)

    - Weight shall be 1.1 lbs. or 500 g.

  - The dome shall conform to these internationally recognized compliance standards:

    - FCC

    - CE

    - UL

    - RoHS

    - C-Tick

    - WEEE

    - REACH