TVD-M2110-2-N

### TruVision 2.0 Mega-Pixel, Lightweight Indoor Dome

- High-performance dome housing

  - The TVD-M2110-2-N shall function as a general-purpose video surveillance dome camera in indoor applications.

  - The TVD-M2110-2-N shall be as manufactured by Interlogix or an approved equal.

  - The camera housing shall be constructed of an aluminum base with an acrylic dome.

    - Light weight metal base, acrylic dome.

    - The housing shall utilize a three-axis gimbal to allow for optimization of the field of view in various mounting locations.

    - The housing shall include a secondary BNC analog video output on the rear of the camera for local test monitor during setup.

  - The camera shall meet or exceed the following specifications:

    - The sensor shall be a Progressive Scan, 1/3-inch format, 2.0 Mega-Pixel CMOS image sensor.

      - Effective Pixels (H xV): 1600 × 1200, 2 MPX

      - Minimum FPS at 1600x1200 resolution to be 10 FPS

      - 1280 x 720p at 15 FPS minimum.

    - Real-time video streaming (30 FPS) at 704 x 480 minimum resolution.

    - Lens options: 2.8 to 11 mm, F1.4 / Manual Iris lens

    - The camera shall have an NTSC composite video output.

      - The video output shall be 1.0 V p-p at 75-ohm load

    - The composite video output shall be DC internal synchronization.

    - The unit will provide dual IP streams for managing bandwidth usage.

    - The sensitivity shall be 0.68 Lux @ F1.4; 0.13 Lux @ F1.4 w/ sense up × 5

    - The signal-to-noise ratio shall be better than 50 dB.

    - Exposure/Shutter to be ¼ second to 1/100,000 second.

    - The camera shall have a digital day night mode.

    - The camera shall have Auto color balance and white balance as AWB or ATB.

    - The camera shall offer two-way audio channel.

      - Compression – OggVorbis, 16 kBit/s

      - Input: 1 channel 3.5 mm audio interface (2.0 - 2.4 Vp-p, 1 kOhm)

      - Output: 1 channel 3.5 mm audio interface (Line level, 600 Ohm)

    - Camera shall have a Media SDHC card slot, 16 GB capacity for recording

      - Edge Recording capable

    - The camera shall have a varifocal lens

    - Intelligence in the Camera

      - Motion Detection: On-board, 396 zones per camera (22 x 18 grid)

      - Privacy Masking: On-board, 396 zones per camera (22 x 18 grid)

      - Health Check: On-board

18.    Camera shall have Alarm Handling:

      - Alarm Input: 1,  Alarm Output: Form C alarm relay (1)

  - Network:

    - Networkable via Ethernet (TCP/IP) for remote functions

    - Dual Streaming: Main Stream and Sub Stream up to real-time CIF

    - Protocol: TCP/IP, HTTP, DHCP, DNS, RTP/RTCP, PPPoE

    - Type: 1 RJ45 10M / 100M self-adaptive Ethernet port

    - Compression: H.264 or MPEG4

  - The electrical specifications for the dome shall be as follows:

    - The camera shall have a universal input voltage of 12 VDC or 24 VAC, POE.

    - Power consumption shall be 4 watts max.

  - The environmental specifications for the dome shall be as follows:

    - Operating temperature shall be -10 to 60 degrees Celsius (14 to 140 °F).

  - The physical specifications for the dome shall be as follows:

    - Dimensions shall be 145 × 133 mm (5.75" × 5.26").

    - Weight shall be 900 g (1.98 lbs).

  - The dome shall conform to these internationally recognized compliance standards:

    - FCC

    - CE

    - UL

    - RoHS