TVD-N210V-2

### TruVision VGA, Dome, Vandal Resistant High-Performance Camera

  - The TVD-N210V-2 dome shall function as a general-purpose video surveillance camera in indoor applications.

  - The domes shall be as manufactured by Interlogix or an approved equal.

  - The camera housing shall be constructed of heavy-duty aluminum with a polycarbonate dome.

    - The housing shall have a vandal rating up to IK10 (IEC60068-2-75Eh,50J; EN50102).

    - The housing shall utilize a three-axis gimbal to allow for optimization of the field of view in various mounting locations.

    - The housing shall include a secondary analog video output inside the dome bubble for local test monitor during setup.

  - The camera shall meet or exceed the following specifications:

    - The sensor shall be a 1/4-inch format, progressive scan CMOS image sensor.

      - Maximum Resolution (H x V): 640 x 480 (30 FPS)

    - Lens:  2.8-11 @ F1.4 /Auto Iris lens

    - The camera shall have an NTSC or PAL composite video output (analog format based on model).

      - The video output shall be 1.0 V p-p at 75-ohm load

    - The composite video output shall be DC internal synchronization.

    - The unit will provide dual IP streams for managing bandwidth usage.

      - Main stream shall be a configurable H.264 or MPEG4 stream.

      - Sub stream shall be a configurable H.264, MPEG4 or MJPEG stream.

    - All models shall feature a day/night mode

      - TVD-N210V-2 models shall feature true day night with auto removable IR cut filter.

    - The sensitivity shall be 0.03 Lux (color mode).

    - The signal-to-noise ratio shall be better than 50 dB.

    - Exposure/Shutter to be ¼ second to 1/100,000 second.

    - The camera shall have Auto color balance and white balance as AWB.

    - The camera shall offer two-way audio channel.

      - Compression – OggVorbis, 16 kBit/s

      - Input: 1 channel 3.5 mm audio interface (2.0 - 2.4 Vp-p, 1 kOhm)

      - Output: 1 channel 3.5 mm audio interface (Line level, 600 Ohm)

    - Camera shall have a Media SDHC card slot, up to 32 GB capacity for recording

      - Edge Recording capable via schedule, motion detection or alarm

      - Log file recording

    - Intelligence in the Camera

      - Motion Detection: On-board, 396 zones per camera (22 x 18 grid)

      - Privacy Masking: On-board, 396 zones per camera (22 x 18 grid)

    - Camera shall have Alarm Handling:

      - Alarm Input: 2,  Alarm Output: 2 Form C alarm relay

  - Communication Protocols

    - The camera shall support PSIA and ONVIF communication and streaming standards.

    - The camera shall support CGI/HTTP command set.

  - Network:

    - Networkable via Ethernet (TCP/IP) for remote functions

    - Dual Streaming: Configurable Main Stream and Sub Stream

    - Adjustable variable or constant bitrates from 32 Kbps to 8Mbps

    - Protocol: TCP/IP, HTTP, DHCP, DNS, RTP/RTCP, PPPoE

    - Type: 1 RJ45 10M / 100M self-adaptive Ethernet port

    - Compression: H.264/MPEG4 (main) or H.264/MJPEG/MPEG4 (sub stream)

  - The electrical specifications for the dome shall be as follows:

    - The camera shall have an input voltage of 12 VDC or POE (802.3af).

    - Power consumption shall be 4.5 watts max.

  - The environmental specifications for the dome shall be as follows:

    - Operating temperature shall be -10 to 60 degrees Celsius.

  - The physical specifications for the dome shall be as follows:

    - Dimensions shall be 69 x 63x 158mm (2.7 x 2.4 x 6.3 in.)

    - Weight shall be 2.8 lbs. or 1300 g.

  - The dome shall conform to these internationally recognized compliance standards:

    - FCC

    - CE

    - UL

    - RoHS

    - C-Tick

    - WEEE

    - REACH