### CyberDome II Integrated PTZ Camera

- Integrated pan/tilt/zoom (PTZ) dome receiver with options for camera, housing, and bubble.

  - The CyberDome II camera module, housing, and pan/tilt driver unit shall be as manufactured by GE Security or an approved equal.

  - The high-speed pan/tilt drive unit shall meet or exceed the following design and performance specifications:

    - The CyberDome II shall use a menu-based system for programming the camera and pan/tilt unit.

      - A compatible keypad or other control device can be used to navigate through the menus.

      - The menus shall have the following options:

        - Setup

          - Clear functions

          - Auto slow shutter

            - The auto slow shutter feature shall slow down the shutter speed of the camera to produce better images in dark situations.

          - Autopan speed

            - The user shall have a choice of slow, medium, or fast.

          - Auto return

            - The auto return feature shall allow the user to set the camera after a period of inactivity to a set position, preset, tour, or autopan mode.

            - The minimum return time shall be 15 seconds.

            - The maximum return time shall be 60 minutes.

          - Azimuth indicator

            - The indicator shall enable a directional icon to be displayed on the screen.

          - Day/night switch

            - The switch shall be available only on day/night model cameras.

            - The switch shall allow the user to set the level at which the camera’s infrared filter automatically switches on at dusk and off at dawn.

          - Electronic zoom

            - The electronic zoom shall expand the camera’s ability to digitally zoom in.

            - The factory default setting for most camera modules is 4x.

          - Exposure control

            - The exposure control shall allow the user to set the method the camera uses to adjust to different light levels.

          - Image freeze

          - Lens initialization

          - Lift/gain

          - Max tilt/pan speed

          - Preset speed

          - Proportional speed

            - The proportional speed feature shall decrease the pan/tilt speed of the camera module to provide better tracking when the camera is zoomed in.

          - Quick spin

            - The quick spin feature shall allow the camera to automatically rotate 180 degrees when the bottom tilt limit is reached, so that movement directly under the camera can be seen.

          - Shutter speed

          - Site number display

          - Synchronization

          - Temperature display

          - White balance

          - Zoom speed control

        - Presets

        - Titles/areas

          - Preset titles

            - Each preset shall be able to display a 24-character title.

          - Area titles

            - Each area shall be able to display a 24-character title.

          - Camera titles

            - Each camera shall be able to display a 24-character title.

          - Tour titles

            - Each preset tour and Shadow Tour shall be able to display a 24-character title.

        - Current status

        - Tours

          - Preset tours

            - A preset tour shall be a group of sequential preset positions.

            - Each preset tour shall have up to 64 steps.

          - Shadow Tours

            - A Shadow Tour shall be a continuous tour that the camera learns by recording the user’s manual operation of the camera via a keypad or other control device.

            - Each Shadow Tour shall last up to four minutes.

            - Up to four Shadow Tours shall be possible with each CyberDome II unit.

        - Privacy masks

        - System startup

          - Passwords

          - Installation date

          - Power on action

            - The power on action shall allow the user to set a specific operation to occur on start up.

          - Position reporting

    - The CyberDome II shall be compatible with the following protocols for twisted pair data transmission:

      - Digiplex RS422

      - Impac RS485

      - ASCII RS422

      - Pelco D

      - Pelco P

      - Ultrak

      - VCL

      - Vicon

      - AD Manchester

      - AD 4800

      - Panasonic

      - Bosch RS232

      - Bosch Biphase

      - KD6

    - The unit shall allow passcode protection that prevents unauthorized personnel from changing programming settings while still allowing operator access.

    - Compatible units shall have built-in day/night control through both the camera module and through a light sensor that has multiple thresholds.

    - The unit shall have variable speed pan/tilt with up to 32 speeds.

    - The unit shall operate between 0.5 and 200 degrees in manual pan.

    - The unit shall operate between 0.5 and 120 degrees per second in manual tilt.

    - The maximum pan/tilt speed shall be 340 degrees/second.

    - Each PTZ shall provide DIP switches for setting the camera’s site address and communication protocol.

  - The available cameras shall meet or exceed the following specifications:

    - The camera shall have the form factor as typical of a CCTV video camera for use in a dome housing.

    - The signal system shall be available in NTSC or PAL video signal format.

    - The camera shall provide a composite video output of 1.0 V p-p.

    - The image-capturing device shall be a 1/4-inch CCD image sensor.

    - The camera shall provide a horizontal resolution of:

      - 470 TV lines, NTSC

      - 460 TV lines, PAL

    - The camera scanning system shall be 2:1 interlacing.

    - The camera shall have digital signal processing.

    - The camera shall have automatic white balance.

    - The camera shall also be capable of manual setup for white balance via the menu system.

    - The camera shall have automatic iris control and manual override.

    - The camera shall provide backlight compensation while in automatic iris control mode.

    - The following camera options shall be available with the CyberDome II dome:

      - 22x color

        - Image Zoom: Optical 22x

        - Lens: F=4 mm (wide) to 64 mm (tele); F1.6

        - Angle of view (H): 47.3° (wide) to 2.2° (tele)

        - Sensitivity: 3 lux

        - Electronic shutter speed: 1/60 to 1/30,000 sec, NTSC; 1/50 to 1/30,000 sec, PAL

      - 18x day/night

        - Zoom: Optical 18x, digital 12x

        - Lens: F=4.1 mm (wide) to 73.8 mm (tele); F1.4 to F3.0

        - Angle of view (H): 48° (wide) to 2.8° (tele)

        - Sensitivity: 0.7 lux at 1/60 sec NTSC or 1/50 sec PAL; 0.05 lux at 1/4 sec NTSC or 1/3 sec PAL

        - Electronic shutter speed: 1/1 to 1/10,000 sec, 14 steps, NTSC; 1/1 to 1/10,000 sec, 14 steps, PAL

      - 23x day/night w/ WDR

        - Zoom: Optical 23x, digital 10x

        - Lens: F=3.6 mm (wide) to 82.8 mm (tele); F1.6 to F3.7

        - Angle of view (H): 54° (wide) to 2.5° (tele)

        - Sensitivity: 0.2 lux at 1/4 sec NTSC or 1/3 sec PAL

        - Sensitivity, ICR-on: 3 lux at 1/60 sec NTSC or 1/50 sec PAL; 0.2 lux at 1/4 sec NTSC or 1/3 sec PAL

        - Electronic shutter speed: 1/60 to 1/4 ksec, NTSC; 1/50 to 1/4 ksec, PAL

      - 26x day/night

        - Zoom: Optical 26x, digital 12x

        - Lens: F=3.5 mm (wide) to 91 mm (tele); F1.6 to F3.8

        - Angle of view (H): 54.2° (wide) to 2.2° (tele)

        - Sensitivity: 1.0 lux at 1/60 sec NTSC or 1/50 sec PAL; 0.07 lux at 1/4 sec NTSC or 1/3 sec PAL

        - Sensitivity, ICR-on: 0.15 lux at 1/60 sec NTSC or 1/50 sec PAL; 0.01 lux at 1/4 sec NTSC or 1/3 sec PAL

        - Electronic shutter speed: 1/1 to 1/10,000 sec, 14 steps, NTSC; 1/1 to 1/10,000 sec, 14 steps, PAL

      - 36x day/night

        - Zoom: Optical 36x, digital 12x

        - Lens: F=3.4 mm (wide) to 122.4 mm (tele); F1.6 to F4.5

        - Angle of view (H): 57.8° (wide) to 1.7° (tele)

        - Sensitivity: 1.4 lux at 1/60 sec NTSC or 1/50 sec PAL; 0.1 lux at 1/4 sec NTSC or 1/3 sec PAL

        - Sensitivity, B&W mode, ICR-off: 0.01 lux at 1/4 sec NTSC or 1/3 sec PAL

        - Electronic shutter speed: 1/4 to 1/10,000 sec, 20 steps, NTSC; 1/3 to 1/10,000 sec, 20 steps, PAL

        - Resolution: 530 TVL NTSC, 530 TVL PAL

    - All available CyberDome II dome housings shall meet or exceed the following specifications:

      - Standard housings shall include both coaxial and UTP video transmission connections.  Select housing versions will include only coaxial video connections

        - The connectors for the housing shall be as follows:

          - Coaxial

            - Video: BNC

            - Control: Removable terminal strip

            - Power input: Two-position removable terminal strip

          - UTP

            - Video: Twisted-pair using removable terminal strip

            - Control: Removable terminal strip

            - Power input: Two-position terminal strip

      - Each housing shall have site-tied memory so that cameras can be removed and switched between housings, yet the settings for a specific location shall remain the same.

      - Standard housings shall have the following control protocols:

        - Input signal

          - BBV

          - Coaxitron

    - The following housing options shall be available:

      - Flush-mount housing

        - The housing shall be used indoors only.

        - The housing shall be designed for in-ceiling applications.

        - The housing shall include a T-bar support kit.

        - The housing shall have an environmental rating of IP50.

        - The housing shall operate at temperatures between 32 and 122°F (0 to 50°C).

          - Maximum temperature is 140°F (60°C) for 22X and 23X models.

        - The housing shall be constructed of ABS plastic.

        - The electrical specifications for the housing shall be as follows:

          - Operating voltage: 24 VAC ± 4 VAC

          - Power, startup: 25 VA (less than 0.5 sec)

          - Power, running: 20 VA

        - The physical specifications for the housing shall be as follows:

          - Dimensions: 9.5 in. diameter (241 mm) at its widest point by 9.6 in. high (243 mm).

        - Weight (including housing, bubble, and pan/tilt unit): 6 lb., 8.5 oz. (2.96 kg).

      - Pendant-mount housing

        - The housing shall be weatherproof for indoor and outdoor applications.

        - The housing shall be available with an optional heater unit.

        - The housing shall have an environmental rating of IP66.

        - The housing shall operate at temperatures between 32 and 122°F (0 to 50°C) without a heater/fan unit or between -40 and 122°F (-40 and 50°C) with a heater unit.

          - Maximum temperature is 140°F (60°C) for 22X and 23X models.

        - The housing shall have a gray finish.

        - The electrical specifications for the housing shall be as follows:

          - Operating voltage: 24 VAC ± 4 VAC

          - Power, startup: 25 VA (less than 0.5 sec)

          - Power, running: 20 VA

          - Power, startup, with heater and fan unit: 93 VA

          - Power, running, with heater and fan unit: 45 VA

        - The physical specifications for the housing shall be as follows:

          - Dimensions: 9.5 in. diameter (241 mm) at its widest point by 11.9 in. high (303 mm).

          - Weight (including housing, heater/fan unit, bubble, and pan/tilt unit): 6 lb., 8.5 oz. (2.96 kg).

      - Wall-mount housing

        - The housing shall be weatherproof for indoor and outdoor applications.

        - The housing’s wall arm shall be constructed of cast aluminum.

        - The housing shall be available with an optional heater unit.

        - The housing shall have an environmental rating of IP66.

        - The housing shall operate at temperatures between 32 and 122°F (0 to 50°C) without a heater unit or between -40 and 122°F (-40 and 50°C) with a heater unit.

          - Maximum temperature is 140°F (60°C) for 22X and 23X models.

        - The housing shall have a gray finish.

        - The electrical specifications for the housing shall be as follows:

          - Operating voltage: 24 VAC ± 4 VAC

          - Power, startup: 25 VA (less than 0.5 sec)

          - Power, running: 20 VA

          - Power, startup, with heater and fan unit: 93 VA (less than 0.5 sec)

          - Power, running, with heater and fan unit 45 VA

        - The physical specifications for the housing shall be as follows:

          - Dimensions: 9.5 in. diameter (241 mm) at its widest point by 9.6 in. high (243 mm).

          - Weight (including housing, heater/fan unit, bubble, and pan/tilt unit): 8 lb., 8.5 oz. (3.87 kg).

      - Heavy-duty housing

        - The housing shall be weatherproof and designed to hold up under tough outdoor use.

        - The housing shall come with adjustable wall-mount brackets.

        - The housing shall be available with an optional heater unit.

        - The housing shall operate best at temperatures between 32 and 122°F (0 to 50°C) without a heater unit or between -13 and 122°F (-25 and 50°C) with a heater unit.

        - Maximum temperature is 140°F (60°C) for 22X and 23X models.

        - The housing shall be constructed of 1/4–inch aluminum.

        - The housing shall have a gray finish.

        - The electrical specifications for the housing shall be as follows:

          - Operating voltage: 24 VAC ± 4 VAC

          - Power, startup: 25 VA (less than 0.5 sec)

          - Power, running: 20 VA

          - Power, startup, with heater and fan unit: 153 VA (less than 0.5 sec)

          - Power, running, with heater and fan unit: 66 VA

        - The physical specifications for the housing shall be as follows:

          - Dimensions: 10.56 in. (268 mm) wide by 9.29 in. high (236 mm).

          - Weight (including housing, heater/fan unit, bubble, and pan/tilt unit): 13 lb., 8 oz. (6.12 kg).

      - Rugged pendant-mount housing

        - The housing shall be weatherproof and designed for outdoor applications that require tamper resistance.

        - The housing shall be available with an optional heater unit.

        - The housing shall have an environmental rating of IP66.

        - The housing shall operate best at temperatures between 32 and 122°F (0 to 50°C) without a heater unit or between -40 and 122°F (-40 and 50°C) with a heater unit.

          - Maximum temperature is 140°F (60°C) for 22X and 23X models.

        - The housing shall be constructed of cast aluminum.

        - The housing shall have a gray finish.

        - The electrical specifications for the housing shall be as follows:

          - Operating voltage: 24 VAC ± 4 VAC

          - Power, startup: 25 VA (less than 0.5 sec)

          - Power, running: 20 VA

          - Power, startup, with heater unit: 153 VA (less than 0.5 sec)

          - Power, running, with heater unit: 66 VA

        - The physical specifications for the housing shall be as follows:

          - Dimensions: 9.5 in. diameter (241 mm) at its widest point by 11.1 inches high (283 mm).

          - Weight (including housing, heater/fan unit, bubble, and pan/tilt unit): 12 lb., 8.5 oz. (5.68 kg).

    - The CyberDome II dome bubbles shall meet or exceed the following specifications:

      - The bubble shall be available in the following:

        - Clear acrylic (no light loss)

        - Smoke acrylic (approximately 1/2 f-stop light loss)

        - Chrome mirrored acrylic (approximately 2 f-stop light loss)

        - Gold mirrored acrylic (approximately 2 f-stop light loss)

        - Polycarbonate clear (no light loss)

          - The polycarbonate clear bubble shall be available for heavy duty and rugged housings only.

        - Polycarbonate smoke (approximately 1/2 f-stop light loss)

          - The polycarbonate smoke bubble shall be available for heavy duty and rugged housings only.

          - Each bubble shall include interlocking clips and safety cables, which vary depending on bubble and housing.

    - The CyberDome II dome shall have a variety of optional mounting accessories available.

      - The cast-aluminum wall-mount arm shall be used to mount a dome to a vertical surface.

        - The wall-mount arm shall be for indoor or outdoor use.

        - The wall-mount arm shall be usable with both the cast aluminum outdoor rugged pendant housing and the plastic indoor and outdoor pendant housing.

        - The arm shall require a minimum load of 100 pounds when used with the rugged pendant housing.

        - The arm shall require a minimum load of 50 pounds when used with the plastic pendant housing.

        - The wall-mount arm shall be capable of being attached directly to a vertical surface or mated with a corner-mount, pole-mount, or roof-mount bracket.

        - The wall-mount arm shall have the following additional specifications:

          - Weight: 2.0 lb. (0.91 kg)

          - Construction: Aluminum

          - Finish: Gray

      - The parapet swing-arm mount shall be used to mount a dome to a roof parapet.

        - The parapet swing-arm mount shall be able to swing the dome out from a building and in above a roof for a broad range of surveillance coverage and easier servicing.

        - The parapet swing-arm mount shall require a minimum load of 100 pounds when used with any CyberDome II housing.

        - The parapet swing-arm mount shall have the following additional specifications:

          - Weight: 7.75 lb. (3.52 kg)

          - Construction: Aluminum

          - Finish: Gray powder coat

      - The corner-mount adapter shall be used to mount a dome to the corner of a building.

        - The corner-mount adapter shall be usable with a wall-mount arm or to suspend the dome.

        - The adapter shall also be able to mount a power supply.

        - The adapter shall require a minimum load of 100 pounds when used with heavy-duty or rugged housings and a minimum load of 50 pounds when used with all other housings.

        - The adapter shall have the following additional specifications:

          - Weight: 1.9 lb. (0.86 kg)

          - Construction: Aluminum

          - Finish: Gray powder coat

      - The pole-mount adapter shall be used to mount a dome to a pole.

        - The pole-mount adapter can be attached to the pole using bolts or three adjustable stainless-steel bands, not included with the adapter.

        - The adapter shall be usable with a wall-mount arm or gooseneck wall-mount arm to suspend the dome.

        - The adapter can also be used to mount a power supply.

        - The adapter shall require a minimum load of 100 pounds when used with heavy-duty or rugged housings and a minimum load of 50 pounds when used with all other housings.

        - The adapter shall have the following additional specifications:

          - Weight: 14.4 oz. (408 g)

          - Construction: Aluminum

          - Finish: Gray powder coat

      - The roof-mount adapter shall be used to hinge a dome back over a roof for safe servicing.

        - The roof-mount adapter shall be usable with a wall-mount arm to suspend the dome.

        - The adapter shall require a minimum load of 100 pounds when used with heavy-duty or rugged housings and a minimum load of 50 pounds when used with all other housings.

        - The adapter shall have the following additional specifications:

          - Weight: 1.2 lb. (0.54 kg)

          - Construction: Aluminum

          - Finish: Gray powder coat

      - The T-bar ceiling panel shall be used to install flush-mount housings into paneled T-bar type ceilings, which consist of a grid of metal T-bars supporting removable panels.

        - Each T-bar ceiling panel shall measure 24 x 24 in. to replace one 24 x 24 in. removable ceiling panel or half of one 24 x 48 in. removable ceiling panel.

        - The panel shall require a minimum load of 50 pounds when used with the flush-mount housing.

        - The panel shall have the following additional specifications:

          - Weight: 3.2 oz. (91 g)

          - Construction: Steel

          - Finish: White enamel

      - The T-bar support kit shall be used to install flush-mount housings into paneled T-bar type ceilings, which consist of a grid of metal T-bars supporting removable panels.

        - The T-bar support kit shall include one ceiling ring, two angle brackets, and mounting hardware.

        - The kit shall distribute the weight of the dome between the T-bars of the ceiling for greater stability.

        - The kit shall require a minimum load of 50 pounds when used with the flush-mount housing.

        - The kit shall have the following additional specifications:

          - Weight: 6.4 oz. (181 g)

          - Construction: Steel

          - Finish: Galvanized

      - The corner parapet-capping brackets shall be used to mount a dome to a roof parapet.

        - The brackets shall provide a platform for attaching a roof-mount adapter.

        - The narrow corner parapet-capping bracket shall fit 6 to 12 in. (152 to 305 mm) parapets.

        - The wide corner parapet-capping bracket shall fit 12 to 17 in. (305 to 432 mm) parapets.

        - The narrow corner parapet-capping bracket shall weigh 7 lb. (3.18 kg).

        - The wide corner parapet-capping bracket shall weigh 8 lb. (3.63 kg).

      - The straight parapet-capping brackets shall be used to mount a dome to a roof parapet.

        - The brackets shall provide a platform for attaching a roof-mount adapter.

        - The narrow straight parapet-capping bracket shall fit 6 to 12 in. (152 to 305 mm) parapets.

        - The wide straight parapet-capping bracket shall fit 12 to 17 in. (305 to 432 mm) parapets.

        - The narrow straight parapet-capping bracket shall weigh 14 lb. (6.4 kg).

        - The wide straight parapet-capping bracket shall weigh 15 lb. (6.8 kg).

  - The camera and pan/tilt unit shall conform to these internationally recognized compliance standards:

    - FCC

    - CE

    - C-Tick

    - UL

    - RoHS