DIVISION 27 21 29

IFS 8 PORT GIGABIT SWITCH WITH 4 POE PORTS

ES3001-4P-4T

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- Eight 10/100/1000Mbps TP Port with four PoE-AF Gigabit Switch

1.02 SECTION INCLUDES

- IFS ES3001-4P-4T 8-Port Gigabit Switch with 4 PoE Ports– Standalone

1.03 REFERENCES

- Federal Communications Commission (FCC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provide 8 10/100/1000Base-T copper ports with 4 IEEE 802.3af Power over Ethernet Injector

- The system shall utilize EIA568, category 5/5e/6, 4-pair cables for 10Base-T or 100Base-TX and 1000Base-T to transfer Ethernet data and 48V DC power simultaneously.

1.05 SUBMITTALS

- Manufacturer’s Installation and Operating Manual: Printed installation and operating information for the unmanaged PoE Switch.

- Warranty: Manufacturer’s Printed Warranty.

1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment.

- Storage Temperature not to exceed: -20˚ C to +70˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Products shall operate in an environment with an ambient temperature range of 0˚ C to +50˚ C with the assistance of fan-forced cooling.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 5% to 95% (non-condensing).

1.08 WARRANTY

- Standard UTC Fire & Security Inc. Comprehensive Warranty: UTC Fire & Security warrants the product to be free of factory defects under manufacture’s 3 Years Warranty.

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer:

- IFS Brand

UTC Fire & Security, Inc.

8985 Town Center Parkway

Bradenton, FL 34202-5129

- Phone 1-855-286-8889

- Email: presales@interlogix.com

- Substitutions: Not Permitted

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

2.03 GENERAL SPECIFICATIONS

- The unmanaged PoE Switch shall be an ES3001-4P-4T model.

- The switch features 8 fixed 10/100/1000T electrical ports

- The switch shall support the Ethernet data IEEE 802.3 protocol using Auto-negotiating and Auto-MDI/MDI-X features.

- The switch shall provide power, fan alert, speed, link / act status and PoE In-use status indicating LED’s for monitoring proper system operation.

- The switch shall be a 1U (one U, 1.75 inches) 10-inch equipment.

- The switch shall be connected with EIA568A/B Cat 5/5e/6 UTP/STP cable system for its RJ-45 interface ports.

- The ES3001-4P-4T shall comply with IEEE 802.3af Power over Ethernet:

- The ES3001-4P-4T shall support IEEE 802.3af Power over Ethernet detection and 48VDC power injection at port#1 to port#4.

- The ES3001-4P-4T is also the power injectors which transmit DC Voltage to the Cat5/5e/6 cable and transfer data and power simultaneously to remote PD (Powered Device) equipments.

- The ES3001-4P-4T shall Auto-detect of PoE IEEE 802.3af equipment; protect devices from being damaged by incorrect installation.

- The ES3001-4P-4T shall support total distance up to 100 meters on PoE ports.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3/3u/3ab

- Data Rate: 10/100/1000 Mbps

- Data Inputs: 8

- Operation Mode: Simplex or Duplex

2.05 STATUS INDICATORS

- System

| 1. | PWR FAN Alert | Green Orange | The switch unit is power on Lights to indicate that the FAN failure. |
| --- | --- | --- | --- |

B. 10/100/1000Base-T Interfaces

| 1. | Speed | Green | Lit: indicate the switch is successfully connecting to the network at 1000Mbps. Off: indicate that the switch is successfully connecting to the network at 10Mbps or 100Mbps. |
| --- | --- | --- | --- |
| 2. | LNK/ ACT | Green | Lights to indicate the link through that port is successfully established. |
| 3. | PoE In-Use | Amber | Blink to indicate that the switch is actively sending or receiving data over that port. Lights to indicate the port is providing 48VDC in-line power. Off to indicate the connected device is not a PoE Powered |

2.06 CONNECTORS

- Power:  Universal AC socket

- Data: RJ-45

2.07 ELECTRICAL SPECIFICATIONS

- Power Characteristics of ES3001-4P-4T:

- Voltage Input:100~240V AC / 50-60Hz

- Current: 1.5A max.

- Power Consumption: Maximum 65Watts with PoE full load

- PoE Output Power of ES3001-4P-4T:

- PoE output budget: 55Watts

- IEEE 802.3af class 3 (15.4W):

2.08 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions: 8.5” x 5.3” x 1.69” (217mm x 135mm x 43mm)

- Finish: Module shall be constructed of a metal enclosure with a powder coat.

- Weight: <1.96 lbs./ 890g

2.09 ENVIRONMENTAL SPECIFICATIONS

- MTBF: > 50,000 Hours

- Operating Temp: 0˚ C to +50˚ C

- Storage Temp: -20˚ C to +70˚ C

- Relative Humidity: 20% to 95% (non-condensing).

2.10 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Federal Communications Commission (FCC) Part 15, Class A

- European Union Compliance (CE) with following standard:

- EN 55022:2006, Class A

- EN61000-3-2:2006

- EN61000-3-3+A2:2005

- EN 55024+A2:2003

2.11 ACCESSORIES

- AC Power cord

- Rubber feet

PART 3 - EXECUTION

3.01 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module.

- The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the installation manual.

3.02 INSTALLATION

- General: Locate fiber optic modules as indicated on the approved detail drawings and install module in compliance with the UTC Fire & Security User’s manual.

3.03 TESTING

- Testing the 10/100/1000T Gigabit Copper Link.

- Verify that the data leads and UTP ports are properly connected.

- Make sure that power is applied to the PoE switch.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

- Testing the 10/100/1000T PoE Copper output capability.

3.04 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all UTP cable scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| IFS PART NO. | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| ES3001-4P-4T | 8 PORT GIGABIT SWITCH WITH 4 POE PORTS | 300 feet (100M) electrical |

## END OF SECTION