DIVISION 27 21 29

IFS POWER OVER COAX – POE- AT SWITCH END

MC252-1T-1CXP

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- One 10/100Mbps TP Port with PoE+ 802.3AT to one Port  BNC Connector

1.02 SECTION INCLUDES

- IFS MC252-1T-1CXP Power over Coax – PoE- at Switch End  – Standalone

1.03 REFERENCES

- Federal Communications Commission (FCC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provide 1 10/100Base-TX copper port with IEEE 802.3at Power over Ethernet Injector and 1 BNC connector for power / data transmit.

- The system shall utilize EIA568, category 5/5e, 4-pair cables for 10Base-T or 100Base-TX to transfer Ethernet data and 44 -57V DC power simultaneously.

- The system shall utilize 5C2V/RG6 75Ωcable.

1.05 SUBMITTALS

- Manufacturer’s Installation and Operating Manual: Printed installation and operating information for the Power over Coax – PoE- at Switch End.

- Warranty: Manufacturer’s Printed Warranty.

1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment.

- Storage Temperature not to exceed: -40˚ C to +75˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Products shall operate in an environment with an ambient temperature range of –40˚ C to +75˚ C without the assistance of fan-forced cooling.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 0% to 95% (non-condensing).

1.08 WARRANTY

- Standard UTC Fire & Security Inc. Comprehensive Warranty: UTC Fire & Security warrants the product to be free of factory defects under manufacture’s 3 Years Warranty.

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer:

- IFS Brand

UTC Fire & Security, Inc.

8985 Town Center Parkway

Bradenton, FL 34202-5129

- Phone 1-855-286-8889

- Email: presales@interlogix.com

- Substitutions: Not Permitted

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

- Model Compatibility Chart: Reference Table B: Product Compatibility Chart

2.03 GENERAL SPECIFICATIONS

- The Power over Coax – PoE- at Switch End shall be an MC252-1T-1CXP model.

- The Power over Coax – PoE- at Switch End with one 10/100Base-TX copper port and one BNC connector system

- The 10/100Base-TX port shall support the Ethernet data IEEE 802.3 protocol using Auto-negotiating and Auto-MDI/MDI-X features.

- The BNC connector supports power and data transmit.

- The module shall provide one Power, TP port 100, Link/Act and PoE status, coaxial VDSL2 interface, Link, CO and CPE, indicating LED’s for monitoring proper system operation.

- The unit also provides a DIP switch for CO/CPE and band plan setting.

- The module shall have DC 52-56V input socket.

- The module shall support UTP distance up to 100 meters

- The module shall be connected with EIA568A/B Cat 5/5e UTP/STP cable system for its RJ-45 interface port

- The MC252-1T-1CXP shall comply with IEEE 802.3at / 802.3af Power over Ethernet:

- The MC252-1T-1CXP shall support total distance up to 100 meters on PoE In port.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3/3u

- Data Rate:

- TP Port: 10/100 Mbps

- BNC Port: BNC, female, Power over Coaxial Output

- Data Inputs: 1

2.05 BNC SPECIFICATIONS

- UTC Fire & Security Model Number MC252-1T-1CXP

- BNC Cable:

- 5C2V/RG6 75Ωcable

- Number of BNC port: 1

- Maximum Distance: 0.62 miles (1km)- 5C2V/RG6 75Ωcable

2.06 STATUS INDICATORS

- System

| 1. | Power | Green | Lit: indicate the system has power. |
| --- | --- | --- | --- |

B. Coaxial / VDSL2 Interfaces

| 1. 2. 3. | LNK CO CPE | Green Green Green | Lit: indicates that the coaxial link is established. Fast Blink: indicates that the coaxial link is at training status (about 10 seconds). Slow Blink: indicates that the coaxial link is at idle status. Lit: indicates the coaxial Bridge is running at CO mode. Lit: indicates the coaxial Bridge is running at CPE mode. |
| --- | --- | --- | --- |

C. 10/100Base-TX Interface

| 1. 2. | 100 LNK/ ACT | Green Green | Lit: indicate that the port is successfully connecting to the network at 100Mbps. Off: indicates the extender is successfully connecting to the network at 10Mbps. Lit: indicate that the port is successfully connecting to the network at 10/100Mbps. Blink: indicate that the converter is actively sending or receiving data over that port. |
| --- | --- | --- | --- |
| 3. | PoE In | Green | Lit: indicates the RJ-45 port is receiving the PoE Power. |

2.07 CONNECTORS

- Power: DC plug female

- Coaxial: BNC, female, Power over Coaxial output

- Ethernet Port: RJ-45

2.08 ELECTRICAL SPECIFICATIONS

- RJ-45 PoE Input: 802.3at/af 44~57V DC

- DC Input: 52~56V DC

2.09 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions: 3.7” x 2.7” x 1.54” (94mm x 70.3 mm x 39.2mm)

- Finish: Module shall be constructed of a metal enclosure with a powder coat.

- Weight:  0.63 lbs./ 288g

2.10 ENVIRONMENTAL SPECIFICATIONS

- MTBF: >100,000 Hours

- Operating Temp: –40˚ C to +75˚ C

- Storage Temp: -40˚ C to +75˚ C

- Relative Humidity: 0% to 95% (non-condensing). If product is installed under condensation conditions, unit shall have conformal coating applied to the printed circuit board.

2.11 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Federal Communications Commission (FCC) Part 15, Class A

- European Union Compliance (CE) with following standard:

- EN 55022:2006, Class A

- EN61000-3-2:2006

- EN61000-3-3+A2:2005

- EN 55024+A2:2003

2.12 ACCESSORIES

- User’s Manual

PART 3 – EXECUTION

3.01PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module. The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the installation manual.

3.02 INSTALLATION

- General: Locate modules as indicated on the approved detail drawings and install module in compliance with the UTC Fire & Security User’s manual.

- The device is used for Point-to-Point connection only (MC252-1T-1CXP to MC252-1P-1CX).

- The BNC connector supports 5C2V/RG6 75Ωcable, the maximum distance is 1km.

3.04 TESTING

- Testing the 10/100TX Fast Ethernet Copper Link.

- Verify that the data leads and UTP ports are properly connected.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

- Testing the 10/100TX PoE Copper input capability.

- Testing the coaxial cable link.

- Power on the MC252-1T-1CXP by connecting its DC power source or 802.3at/af PoE Source.

- Connects coaxial cable between the BNC ports of the MC252-1T-1CXP and MC252-1P-1CX devices.

- Link LED will from blinking to illuminate.

3.05 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all UTP cable scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| IFS PART NO. | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| MC252-1T-1CXP | POWER OVER COAX – POE- AT SWITCH END | 0.62 miles (1km) |

Table B: Product Compatibility Chart

| IFS PART NO. | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| MC252-1P-1CX | POWER OVER COAX-POE-AT IP CAMERA END | 0.62 miles (1km) |

## END OF SECTION