DIVISION 27 21 29

IFS 10/100/1000-T to 1000-X GIGABIT SFP MEDIA CONVERTER

MCR300-1T/1S

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- IFS MCR300-1T/1S 10/100/1000-T to 1000-X SFP Rack Mountable Gigabit Media Converter

1.02 SECTION INCLUDES

- IFS MCR300-1T/1S 10/100/1000-T to 1000-X SFP Gigabit Media Converter – Standalone

1.03 REFERENCES

- Federal Communications Commission (FCC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provides 1 10/100/1000Base-T copper port and 1 1000Base-SX/LX SFP optical multimode or singlemode slot.

- The system shall utilize EIA568, category 5/5e/6, 4-pair cables for 10Base-T or 100Base-TX and 1000Base-T to transfer Ethernet data.

- The system shall utilize 850nm to 1550nm optics capable of 1000Mbps data transmission on multimode / singlemode optical fibers.

1.05 SUBMITTALS

- Manufacturer’s Installation and Operating Manual: Printed installation and operating information for each type of Transmitter/Receiver specified.

- Warranty: Manufacturer’s Printed Warranty.

1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment.

- Storage Temperature not to exceed: -40˚ C to +70˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Product shall operate in an environment with an ambient temperature range of 0˚C to +50˚C without the assistance of fan-forced cooling.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 5% to 90% (non-condensing).

1.08 WARRANTY

- UTCFS Return and Warranty Policy: UTCFS warrants the product to be free of factory defects for period of 3 years.

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer:

- IFS Brand
UTC Fire & Security, Inc.
8985 Town Center Parkway
Bradenton, FL 34202-5129

- Phone 1-855-286-8889

- Email: 	presales@interlogix.com

- Substitutions: Not Permitted

- All fiber optic SFP modules shall be supplied from a single manufacturer.

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

- Model Compatibility Chart: Reference Table B: Product Compatibility Chart

2.03 GENERAL SPECIFICATIONS

- The Gigabit Media Converter with one 10/100/1000Base-T copper port and one 1000Base-SX/LX mini-GBIC slot system shall be an IFS MCR300-1T/1S.

- The 1000Base-SX/LX mini-GBIC slot support 1000Mbps singlemode / multimode fiber

- The 10/100/1000Base-T port shall support the Ethernet data IEEE 802.3 protocol using Auto-negotiating and Auto-MDI/MDI-X features.

- The Gigabit Media Converter shall provide one power, fiber Link / Act, TP port speed and TP port Link / Act LED indicators for proper monitoring of operation.

- The Gigabit mini-GBIC module shall require no in-field electrical or optical adjustments or in-line attenuators.

- The Gigabit Media Converter shall be equipped with one adjustable DIP switch for LFP function setting.

- The module shall support UTP distance up to 100 meters.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3

- Data Rate: 10/100/1000 Mbps

- Data Inputs: 1

- Operation Mode: Simplex or Duplex

2.05 OPTICAL SPECIFICATIONS

- IFS Model Number MCR300-1T/1S.

- Optical Fiber:

- 9/125 micron singlemode

- 62.5/125 micron multimode

- Number of Optical ports: 1

- Number of Fibers Required: 1 or 2, depends on SFP module

- Optical Wavelength: depends on SFP module

- Optical Power Budget: depends on SFP module

- Maximum Distance: depends on SFP module

2.06 STATUS INDICATORS

A. System

| LED | Color | Function |  |
| --- | --- | --- | --- |
| PWR | Green | Lit | Indicates that the device is powered. |

- 10/100/1000Base-T Port

| LED | Color | Function |
| --- | --- | --- |
| TP LNK/ACT | Green | Lit: Indicates that the link through that port is successfully established. |
|  |  | Blink: Indicates that the port is actively sending or receiving data. |
| TP 1000 | Green | Lit: Indicates that port runs in 1000Mbps Full duplex mode. |
|  |  | Off: Indicates that port runs in 10/100Mbps Full duplex mode. |

- 1000Base-SX/LX SFP Slot

| LED | Color | Function |
| --- | --- | --- |
| Fiber LNK/ACT | Green | Lit: Indicates that the link through that fiber port is successfully established. |
|  |  | Blink: Indicates that the fiber port is actively sending or receiving data. |

2.07 CONNECTORS

- Power: DC plug female

- Optical: SFP Slot / LC interface

- Data: RJ-45

2.08 ELECTRICAL SPECIFICATIONS

- Power Adapter: (Not included)

- Input Voltage Range: 100~240VAC

- Output Voltage: 5V DC @ 2A

- The Gigabit Media Converter:

- Input Voltage: 5V DC @ 2A

2.09 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions (WxDxH): 3.82” x 2.76” x 1.02” (97x 70 x 26mm)

- Finish: Module shall be constructed of a metal enclosure.

- Weight < 0.41lbs / 190g

2.10 ENVIRONMENTAL SPECIFICATIONS

- MTBF: >50,000 Hours

- Operating Temp: 0˚ C to +50˚ C

- Storage Temp: -40˚ C to +70˚ C

- Relative Humidity: 5% to 90% (non-condensing).

2.11 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Federal Communications Commission (FCC) Part 15, Class A

- European Union Compliance (CE) with following standard:

- EN 55022:2006, Class A

- EN61000-3-2:2006

- EN61000-3-3+A2:2005

- EN 55024+A2:2003

2.12 ACCESSORIES

- User’s Manual x1

PART 3 - EXECUTION

3.01 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module. The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the user’s manual.

- Optical Fibers

- Caution: NEVER look into the end of an active optical fiber when using laser light output. Eye damage can occur. Wear eye protection when cleaving, terminating, and splicing fiber.

- The number and type – multimode / singlemode of optical fiber shall meet the requirement of the IFS model number.

- All optical fiber cables shall be properly installed and terminated with the mating optical connectors.

- The optical connector on cable shall be cleaned in compliance to optical connector manufactures specifications and covered with dust caps until it is connected to the fiber optic module.

3.02 INSTALLATION

- General: Locate Gigabit Media Converter module as indicated on the approved detail drawings and install module in compliance with the IFS User’s manual.

3.03 TESTING

- Testing the 10/100/1000T Gigabit Ethernet Copper Link.

- Verify that the data leads and UTP ports are properly connected.

- Make sure that power is applied to the Gigabit Media Converter or other equipment used in the system.

- Successful data link operation should be established at this point by communication with connected equipment.

- Testing the 10/100/1000T Copper port capability.

- Testing the Fiber Optic Ethernet Link.

- Verify that the data leads and optical fibers are properly connected.

- Make sure that power is applied to all fiber optic modules, controllers, and receiver drivers or other equipment used in the system.

- Successful data link operation should be established at this point by communication with connected equipment.

3.04 CLEANING

- Follow all instructions for proper use of solvents

and adhesives used for termination and splicing.

- Upon completion of the installation, dispose the

fiber scrap properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| IFS PART NO. | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| MCR300-1T/1S | 10/100/1000-T to 1000-X SFP Rack Mountable Gigabit Media Converter | Dependent on SFP module in use |

* Maximum distance is limited to optical loss of the fiber and any additional loss by connectors, splices and patch panels.

Table B: SFP Transceiver Compatibility Chart

| IFS PART NO. | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| S30-2MLC | SFP-Port 1000Base-SX mini-GBIC module - 220m | 0.14 or 0.34 Miles (220m or 550m) |
| S35-2MLC | SFP-Port 1000Base-SX mini-GBIC module - 220m, Hardened | 0.14 or 0.34 Miles (220m or 550m) |
| S30-2MLC-2 | SFP-Port 1000Base-SX mini-GBIC module – 2km | 1.25 miles (2km) |
| S30-2SLC-10 | SFP-Port 1000Base-LX mini-GBIC module – 10Km | 6.21 Miles (10KM) |
| S35-2SLC-10 | SFP-Port 1000Base-LX mini-GBIC module – 10Km, Hardened | 6.21 Miles (10KM) |
| S30-2SLC-30 | SFP-Port 1000Base-LX mini-GBIC module – 30Km | 18.64 Miles (30KM) |
| S35-2SLC-30 | SFP-Port 1000Base-LX mini-GBIC module – 30Km, Hardened | 18.64 Miles (30KM) |
| S30-2SLC-70 | SFP-Port 1000Base-LX mini-GBIC module – 70Km | 43.49 Miles (70KM) |
| S35-2SLC-70 | SFP-Port 1000Base-LX mini-GBIC module – 70Km, Hardened | 43.49 Miles (70KM) |
| S30-1SLC/A-10 | SFP-Port 1000Base-BX mini-GBIC module A-End (1310/1490nm) – 10Km | 6.21 Miles (10KM) |
| S30-1SLC/B-10 | SFP-Port 1000Base-BX mini-GBIC module B-End (1490/1310 nm) – 10Km | 6.21 Miles (10KM) |
| S30-1SLC/A-20 | SFP-Port 1000Base-BX mini-GBIC module A-End (1310/1490nm) – 20Km | 12.42 Miles (20KM) |
| S30-1SLC/B-20 | SFP-Port 1000Base-BX mini-GBIC module B-End (1490/1310 nm) – 20Km | 12.42 Miles (20KM) |
| S30-1SLC/A-60 | SFP-Port 1000Base-BX mini-GBIC module A-End (1310/1490nm) – 60Km | 37.28 Miles (60KM) |
| S30-1SLC/B-60 | SFP-Port 1000Base-BX mini-GBIC module B-End (1490/1310 nm) – 60Km | 37.28 Miles (60KM) |

* Maximum distance is limited to optical loss of the fiber and any additional loss by connectors, splices and patch panels.

## END OF SECTION