DIVISION 27 21 29

IFS IEEE 802.3AF POWER OVER ETHERNET INJECTOR (MID-SPAN)

MS-POE

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- IFS IEEE 802.3af Power Over Ethernet (PoE) Injector (Mid-Span)

1.02 SECTION INCLUDES

- IFS MS-PoE IEEE 802.3af Power Over Ethernet (PoE) Injector (Mid-Span) – Standalone

1.03 REFERENCES

- Federal Communications Commission (FCC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provide 1 10/100Base-TX copper port with IEEE 802.3af Power over Ethernet Injector.

- The system shall utilize EIA568, category 5/5e, 4-pair cables for 10Base-T or 100Base-TX to transfer Ethernet data and 48V DC power simultaneously.

1.05 SUBMITTALS

- Manufacturer’s Installation and Operating Manual: Printed installation and operating information for the PoE Injector.

- Warranty: Manufacturer’s Printed Warranty.

1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment.

- Storage Temperature not to exceed: -20˚ C to +70˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Product shall operate in an environment with an ambient temperature range of 0 Degree C to +50 Degree C without the assistance of fan-forced cooling.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 5% to 90% (non-condensing).

1.08 WARRANTY

- UTCFS Return and Warranty Policy: UTCFS warrants the product to be free of factory defects for a period of 3 years.

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer:

- IFS Brand
UTC Fire & Security, Inc. 
8985 Town Center Parkway
Bradenton, FL 34202-5129

- Phone 1-855-286-8889

- Email: presales@interlogix.com

- Substitutions: Not Permitted

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

- Model Compatibility Chart: Reference Table B: Product Compatibility Chart

2.03 GENERAL SPECIFICATIONS

- The Power over Ethernet Injector shall be an IFS MS-PoE module.

- The module shall comply with IEEE 802.3af Power over Ethernet.

- The module shall support the Ethernet data IEEE 802.3/802.3u 10/100Base-TX protocol using Auto-negotiating and Auto-MDI/MDI-X features.

- The module features two fixed 10/100TX electrical ports in a compact mini box.

- The port printed with “Ethernet” functions as "Data input".

- The other port on the other side and printed with “Ethernet + DC” functions as "PoE (Data and Power) output".

- The module is also the power injectors which transmit DC Voltage to the Cat5/5e cable and transfer data and power simultaneously between the PSE and PD.

- The module shall Auto-detect of PoE IEEE 802.3af equipment; protect devices from being damaged by incorrect installation.

- The module shall support a total distance of up to 100 meters.

- The module shall provide power and PoE in-use status indicating LED’s for monitoring proper system operation.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.u

- Data Rate: 10/100 Mbps

- Data Inputs: 2

2.05 STATUS INDICATORS

A. LED Definition:

| LED | Color | Function |
| --- | --- | --- |
| Power | Yellow | Illuminates to indicate that the MS-PoE has power. |
| PoE Ready / in-use | Green | Illuminates to indicate the port is providing 48VDC in-line power. |

2.06 CONNECTORS

- Power: DC plug female

- Data:

- Ethernet Port: RJ-45

- Ethernet + DC Port: RJ-45

2.07 ELECTRICAL SPECIFICATIONS

- Power Adapter:

- Input Voltage Range: 100~240VAC

- Output Voltage: 48VDC @ 0.38A

- The PoE Injector:

- Input Voltage: 48VDC

2.08 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions (WxDxH): 2.87” x 2.17” x 0.94” (73 x 55 x 24mm)

- Finish: Module shall be constructed of a plastic enclosure.

- Weight < 0.11lbs / 50g

2.9 ENVIRONMENTAL SPECIFICATIONS

- MTBF: >50,000 Hours

- Operating Temp: 0˚ C to +50˚ C

- Storage Temp: -20˚ C to +70˚ C

- Relative Humidity: 5% to 90% (non-condensing).

2.10 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Federal Communications Commission (FCC) Part 15, Class A

- European Union Compliance (CE) with following standard:

- EN 55022:2006, Class A

- EN61000-3-2:2006

- EN61000-3-3+A2:2005

- EN 55024+A2:2003

- IEC/EN 60950-1

2.11 ACCESSORIES

- AC-to-DC Power Adapter x1

- Power cord x 1 (UK ,AU, AG, IR and SW only)

PART 3 - EXECUTION

3.01 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module. The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the installation manual.

3.02 INSTALLATION

- General: Locate PoE Injector module as indicated on the approved detail drawings and install module in compliance with the IFS User’s manual.

3.03 TESTING

- Testing the 10/100TX Fast Ethernet Copper Link.

- Verify that the data leads and UTP ports are properly connected.

- Make sure that power is applied to the PoE Injector module or other equipment used in the system.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

- Testing the 10/100TX PoE Copper output capability.

3.05 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all UTP cable scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| IFS PART NO. | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| MS-PoE | IEEE 802.3AF POWER OVER ETHERNET INJECTOR (MID-SPAN) | 300 feet (100M) electrical |

* Maximum distance is limited to optical loss of the fiber and any additional loss by connectors, splices and patch panels.

Table B: Product Compatibility Chart

| IFS IEEE 802.3af Power over Ethernet Injector | COMPATIBLE Ethernet Equipment |
| --- | --- |
| MS-PoE | IEEE 802.3u Fast Ethernet standard for 10/100TX protocol IEEE 802.3af Power over Ethernet PD |

## END OF SECTION