DIVISION 27 21 29

NS2503-8P/2C 8-PORT 10/100BASE-TX + 2- GIGABIT TP/SFP MANAGED POE SWITCH

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- IFS NS2503-8P/2C 8 Port Ethernet PoE+   Managed Switch

1.02 SECTION INCLUDES

- IFS NS2503-8P/2C 802.3at Power over Ethernet Managed Switch – Standalone

1.03 REFERENCES

- Underwriters Laboratory (UL)

- Underwriters Laboratory Canada (ULC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- The Layer 2 Managed Switch provides 8 10/100TX ports and 2 Gigabit TP/SFP combination ports delivered in a desktop, compact size case.

- The Gigabit TP / SFP combo ports can either be 1000Base-T for 10 / 100 / 1000Mbps or optical 1000Base-SX / LX through SFP (Small Form-Factor Pluggable) interface.

- The SFP module shall utilize 850nm optics capable of bi-directional data transmission of 1000Base-SX on two multimode optical fibers.

- The SFP module shall utilize 1310nm optics capable of bi-directional data transmission of 1000Base-LX on two single-mode optical fibers.

- The SFP module shall utilize 1310nm/1490nm or 1310nm/1550nm optics capable of bi-directional data transmission of 1000Base-BX on one single-mode optical fiber.

- The NS2503-24P/2C Managed PoE Switch provides 24 10/100Mbps Fast Ethernet ports with 8 802.3at / 802.3af PoE injector, and 2 Gigabit TP/SFP combo interfaces.

- Equipped with the IEEE 802.3at/802.3af compliant PoE injector, the NS2503-8P/2C is able to power up to 8 PoE compliant devices located up to 100 meters (328 ft.) away with the 4-pair Cat 5/5e UTP wire.

1.05 SUBMITTALS

- Manufacturer’s Quick Installation Guide and Operating Manual: Printed installation guide and operating information for the Managed Switch.

1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment. Storage Temperature not to exceed: -20˚ C to +70˚ C

- Deliver materials in unopened factory packaging with Manufacturer’s bar coding to the job site.

- Inspect product upon delivery to assure that specified products have been received.

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements:

- The NS2503-8P/2C shall operate in an environment with an ambient temperature range of 0˚ C to +50˚ C with the assistance of fan-forced cooling.

- Humidity Requirements: Product shall operate in an environment with relative humidity of 20% to 95% (non-condensing).

1.08 WARRANTY

- UTCFS Return and Warranty Policy: UTCFS warrants the product to be free of factory defects for period of 3 years.

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer:

- IFS Brand
UTC Fire & Security, Inc.
8985 Town Center Parkway
Bradenton, FL 34202-5129

- Phone 1-855-286-8889

- Email: 	presales@interlogix.com

- Substitutions: Not Permitted

- All fiber optic modules shall be supplied from a single manufacturer.

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

- Model Compatibility Chart: Reference Table B: Product Compatibility Chart

2.03 GENERAL SPECIFICATIONS

- The Managed Ethernet switch shall be the IFS NS2503-8P/2C model.

- The switch features 8 fixed 10/100TX electrical ports

- The switch features two combination 10/100/1000T electrical ports or 1000SX/LX optical SFP slots.

- The switch shall support the Ethernet data IEEE 802.3 protocol using Auto-negotiating and Auto-MDI/MDI-X features.

- The switch shall provide power, link / act status and PoE In-use status indicating LED’s for monitoring proper system operation.

- The switch shall provide a RS-232 serial connection for local management of the device.

- The switch shall be a 1U (one U, 1.75 inches) 13-inch equipment and it can be installed in standard cabinet or 19-inch rack with rack mount ears.

- The switch shall be connected with EIA568A/B Cat 5/5e/6 UTP/STP cable system on its RJ-45 interface ports.

- The NS2503-8P/2C shall comply with IEEE 802.3at / 802.3af Power over Ethernet standard:

- The NS2503-8P/2C is equipped with power injectors which transmit DC Voltage to the Cat5/5e cable and transfer data and power simultaneously to remote PD (Powered Device) equipments.

- The NS2503-8P/2C shall support IEEE 802.3at Power over Ethernet detection and 52VDC power injection at port#1 to port#8.

- The NS2503-24P/2C shall automatically detect PoE IEEE802.3at / 802.3af equipments and protect devices from being damaged as a result of faulty installation.

- The NS2503-8P/2C shall support total distances of up to 100 meters (328 ft.) on its PoE ports.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3/3u/3ab/3z

- Data Rate:

- Port-1 to Port-8: 10/100 Mbps

- Port-9 and Port-10 TP: 10/100/1000 Mbps

- Port-9 and Port-10 SFP: 1000 Mbps

- Data Inputs: 10

- Operation Mode: Simplex or Duplex

2.05 OPTICAL SPECIFICATIONS

- UTC Fire & Security Model Number NS2503-8P/2C

- Optical Interface: 3.3V SFP (Small Form-Factor Pluggable) slot

- Number of SFP Optical ports:2

- Optical Fiber:

- 62.5/125 micron multimode

- 9/125 micron single-mode

- Number of Fibers Required: 1 or 2, vary on SFP module

- Optical Wavelength:

  - 1000SX multimode: 850nm

  - 1000LX singe-mode: 1310nm

- Optical Power Budget: very on SFP module

- Maximum Distance: 43.49 miles (70km)

2.06 STATUS INDICATORS

- System

| 1. | PWR | Green | The switch unit power is on |
| --- | --- | --- | --- |

B. 10/100Base-TX Interfaces

| 1. 2. | LNK/ ACT PoE In-Use | Green Amber | Illuminates to indicate that the link through that port is successfully established. |
| --- | --- | --- | --- |
|  |  |  | Blinks to indicate that the switch is actively sending or receiving data over that port. Illuminates to indicate that the port is providing 52VDC in-line power. |

C. 1000Base-T / SFP Interfaces

| 1. | 1000 LNK/ ACT | Green | Illuminates to indicate that the link through that port is successfully established with speed 1000Mbps |
| --- | --- | --- | --- |
|  |  |  | Blinks to indicate that the switch is actively sending or receiving data over that port. |
| 2 | 10/100 LNK/ ACT | Green | Illuminates to indicate that the link through that port is successfully established with speed 10Mbps or 100Mbps |
|  |  |  | Blinks to indicate that the switch is actively sending or receiving data over that port. |

2.07 CONNECTORS

- Optical: SFP slot

- Power:  Universal AC socket

- Data: RJ-45

- Console: DB9 Type RS-232 serial com.

2.08 ELECTRICAL SPECIFICATIONS

- Power Characteristics of NS2503-8P/2C:

- Voltage Input:100~240V AC / 50-60Hz

- Current: 1.5A max.

- Power Consumption: Maximum 175Watts with PoE full load

- PoE Output Power of NS2503-8P/2C:

- PoE output budget: 150Watts

- IEEE 802.3at class 3 (30W): Max. 5 ports

- IEEE 802.3af class 3 (15.4W): Max. 8 ports

2.09 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions: 13” x 6.10” x 1.71” / 330 x 155 x 43.5mm, 1U height

- Finish: Module shall be constructed of a metal enclosure with a powder coat.

- Weight: <3.83 lbs./1.74kg

2.10 ENVIRONMENTAL SPECIFICATIONS

- MTBF: > 50,000 Hours

- Operating Temp: 0˚ C to +50˚ C

- Storage Temp: -20˚ C to +70˚ C

- Relative Humidity: 20% to 95% (non-condensing).

2.11 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Number: I.T.E. 6D16

- Federal Communications Commission (FCC) Part 15 subpart B, Class A

- European Union Compliance (CE) with following standard:

- EN 55022:2006+A1:2007, Class A

- EN61000-3-2:2006+A2:2009

- EN61000-3-3:2008

- EN 55024:1998+A1:2001+A2:2003

2.12 ACCESSORIES

- AC Power cord

- Rubber feet

- Rack-mount brackets

- RS-232 DB9 male console cable

PART 3 - EXECUTION

3.01 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module.

- The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the installation manual.

- Rack Mount Module (19” Rack)

- The unit is installed in a standard EIA 19” (482.6 mm) rack or wall standoff bracket adequate for the size and weight of the rack mount unit. The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the user’s manual.

- Optical Fibers

- Caution: NEVER look into the end of an active optical fiber when using laser light output. Eye damage can occur. Wear eye protection when cleaving, terminating, and splicing fiber.

- The number of optical fiber SFP slot shall meet the requirements of the IFS model number.

- All optical fiber cables shall be properly installed and terminated with the mating optical connectors.

- The optical link shall be tested with either a power meter, at a minimum, or OTDR to ensure the link budget (overall path loss) plus an added 3dB of optical safety margin does not exceed the optical power budge.

3.02 INSTALLATION

- General: Locate fiber optic modules as indicated on the approved detail drawings and install module in compliance with the IFS User’s manual.

3.03 TESTING

- Testing the Fiber Optic Ethernet Link.

- Verify that the data leads and optical fibers are properly connected.

- Make sure that power is applied to all fiber optic modules, controllers, and receiver drivers or other equipment used in the system.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

- Testing the 10/100TX Fast Ethernet Copper Link.

- Verify that the data leads and UTP ports are properly connected.

- Make sure that power is applied to the PoE switch.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

- Testing the 10/100TX PoE Copper output capability.

- Make sure that power is applied to the PoE switch.

3.04 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all fiber scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| IFS MODEL | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| NS2503-8P/2C | 8-Port 10/100Base-TX + 2- Gigabit TP/SFP Managed 802.3at PoE Switch | Varies by SFP module |

Table B: Product Compatibility Chart

| IFS SFP Transceiver | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| S30-RJ | SFP-Port SFP100-T mini-GBIC module – 100m | 0.06 Miles (100m) |
| S30-2MLC | SFP-Port 1000Base-SX mini-GBIC module - 220m – Industrial (-40 ~75 °C) | 0.14 or 0.34 Miles (220m or 550m) |
| S30-2MLC-2 | SFP-Port 1000Base-SX mini-GBIC module – 2km | 1.25 miles (2km) |
| S35-2MLC | SFP-Port 1000Base-SX mini-GBIC module - 220m | 0.14 or 0.34 Miles (220m or 550m) |
| S30-2SLC-10 | SFP-Port 1000Base-LX mini-GBIC module – 10Km | 6.21 Miles (10KM) |
| S30-2SLC-30 | SFP-Port 1000Base-LX mini-GBIC module – 30Km | 18.64 Miles (30KM) |
| S30-2SLC-70 | SFP-Port 1000Base-LX mini-GBIC module – 70Km | 43.49 Miles (70KM) |
| S35-2SLC-10 | SFP-Port 1000Base-LX mini-GBIC module – 10Km – Industrial (-40 ~75 °C) | 6.21 Miles (10KM) |
| S35-2SLC-30 | SFP-Port 1000Base-LX mini-GBIC module – 30Km – Industrial (-40 ~75 °C) | 18.64 Miles (30KM) |
| S35-2SLC-70 | SFP-Port 1000Base-LX mini-GBIC module – 70Km – Industrial (-40 ~75 °C) | 43.49 Miles (70KM) |
| S30-1SLC/A-10 | SFP-Port 1000Base-BX mini-GBIC module A-End (1310/1490nm) – 10Km | 6.21 Miles (10KM) |
| S30-1SLC/B-10 | SFP-Port 1000Base-BX mini-GBIC module B-End (1490/1310 nm) – 10Km | 6.21 Miles (10KM) |
| S30-1SLC/A-20 | SFP-Port 1000Base-BX mini-GBIC module A-End (1310/1490nm) – 20Km | 12.42 Miles (20KM) |
| S30-1SLC/B-20 | SFP-Port 1000Base-BX mini-GBIC module B-End (1490/1310 nm) – 20Km | 12.42 Miles (20KM) |
| S30-1SLC/A-60 | SFP-Port 1000Base-BX mini-GBIC module A-End (1310/1490nm) – 60Km | 37.28 Miles (60KM) |
| S30-1SLC/B-60 | SFP-Port 1000Base-BX mini-GBIC module B-End (1490/1310 nm) – 60Km | 37.28 Miles (60KM) |

* Maximum distance is limited to optical loss of the fiber and any additional loss by connectors, splices and patch panels.

## END OF SECTION