DIVISION 27 21 29

POE201-MS/4 IEEE802.3AF 4-PORT POWER OVER ETHERNET INJECTOR HUB

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- IFS POE201-MS/4 Four Port IEEE802.3af Power Over Ethernet  Injector Hub (Mid-Span)

1.02 SECTION INCLUDES

- IFS POE201-MS/4 IEEE802.3af Power over Ethernet Injector Hub (Mid-Span) – Standalone

1.03 REFERENCES

- Federal Communications Commission (FCC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- The IEEE 802.3af Power over Ethernet Injector Hub provides 4-Port "Data" and 4-Port "Data and Power" delivered in a desktop, compact palm size case.

- The PoE in-line power following the standard IEEE 802.3af enables the POE201-MS/4 to power four 802.3af PoE compliant devices at the distances up to 100 meters (328 ft) through the 4-pair Cat 5/5e UTP cable.

- The system shall utilize EIA568, category 5/5e, 4-pair cables for 10Base-T or 100Base-TX to transfer Ethernet data and 48V DC power simultaneously.

1.05 SUBMITTALS

- Manufacturer’s Quick Installation Guide and Operating Manual: Printed installation guide and operating information for the IEEE 802.3af Power over Ethernet Injector Hub.

1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment. Storage Temperature not to exceed: -20˚ C to +70˚ C

- Deliver materials in unopened factory packaging with Manufacturer’s bar coding to the job site.

- Inspect product upon delivery to assure that specified products have been received.

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements:

- The POE201-MS/4 shall operate in an environment with an ambient temperature range of 0˚ C to +50˚ C without the assistance of fan-forced cooling.

- Humidity Requirements: Product shall operate in an environment with relative humidity of 20% to 95% (non-condensing).

1.08 WARRANTY

- UTCFS Return and Warranty Policy: UTCFS warrants the product to be free of factory defects for period of 3 years.

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer:

- IFS Brand

UTC Fire & Security, Inc.

8985 Town Center Parkway

Bradenton, FL 34202-5129

- Phone 1-855-286-8889

- Email: presales@interlogix.com

- Substitutions: Not Permitted

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

- Model Compatibility Chart: Reference Table B: Product Compatibility Chart

2.03 GENERAL SPECIFICATIONS

- The IEEE 802.3af Power over Ethernet Injector Hub shall be an IFS model POE201-MS/4.

- The module shall comply with IEEE 802.3af Power over Ethernet standard.

- The module shall support the Ethernet data IEEE 802.3/802.3u 10/100Base-TX protocol using Auto-negotiating and Auto-MDI/MDI-X features.

- The module features eight fixed 10/100TX electrical ports in a compact size enclosure.

- The port printed with “Data” functions as "Data input".

- The other port on the other side and printed with “Data and Power” functions as "PoE (Data and Power) output".

- The module is equipped with power injectors that transmit DC Voltage to the Cat5/5e cable and transfer data and power simultaneously between the PSE and PD.

- The module shall automatically detect IEEE 802.3af equipment and protect devices from being damaged by faulty installation.

- The module shall support total distance up to 100 meters (328 ft).

- The module shall provide power, and PoE ready / in-use status indicator LED’s for monitoring proper system operation.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3/3u

- Data Rate:

- Port-1 to Port-4: 10/100 Mbps

- Data Inputs: 4

- Operation Mode: Simplex or Duplex

2.05 STATUS INDICATORS

- System

| 1. | PWR | Green | The PoE Hub unit is power on |
| --- | --- | --- | --- |

B. 10/100Base-TX Interfaces

| 1. | PoE In-Use | Green | Lights to indicate the port is providing 48VDC in-line power. Off to indicate the connected device is not a PoE Powered |
| --- | --- | --- | --- |

2.06 CONNECTORS

- Power: DC plug female x1

- Data:

- Ethernet Port: RJ-45 x4

- Ethernet + DC Port: RJ-45 x4

2.07 ELECTRICAL SPECIFICATIONS

- Power Adapter: (not included)

- Input Voltage Range: 100~240VAC

- Output Voltage: 48VDC @ 1.5A

- The PoE Injector:

- Input Voltage: 48VDC

2.08 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions (WxDxH): 3.82” x 2.76” x 1.02” (97x 70 x 26mm)

- Finish: Module shall be constructed of a metal enclosure.

- Weight: 0.48lbs./220g

2.09 ENVIRONMENTAL SPECIFICATIONS

- MTBF: > 50,000 Hours

- Operating Temp: 0˚ C to +50˚ C

- Storage Temp: -20˚ C to +70˚ C

- Relative Humidity: 20% to 95% (non-condensing).

2.10 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Federal Communications Commission (FCC) Part 15, Class A

- European Union Compliance (CE) with following standard:

- EN 55022:2006, Class A

- EN61000-3-2:2006

- EN61000-3-3+A2:2005

- EN 55024+A2:2003

2.11 ACCESSORIES

- AC-DC Power Adapter

- AC Power cord

PART 3 - EXECUTION

3.01 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module.

- The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the installation manual.

3.02 INSTALLATION

- General: Locate PoE Injector module as indicated on the approved detail drawings and install module in compliance with the IFS User’s manual.

3.03 TESTING

- Testing the 10/100TX Fast Ethernet Copper Link.

- Verify that the data leads and UTP ports are properly connected.

- Make sure that power is applied to the PoE Injector module or other equipment used in the system.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

- Testing the 10/100TX PoE Copper output capability.

- Make sure that power is applied to the PoE switch.

3.04 CLEANING

- Follow all instructions for proper connector termination.

- At completion of the installation, dispose of all UTP cable scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| IFS MODEL | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| POE201-MS/4 | IEEE802.3af 4-Port Power over Ethernet Injector Hub (Mid-Span) | 100 Meters (328 feet) electrical |

Table B: Product Compatibility Chart

| IFS MODEL | COMPATIBLE EHTERNET EQUIPMENT |
| --- | --- |
| POE201-MS/4 - IEEE 802.3af Power over Ethernet Injector | IEEE 802.3u Fast Ethernet standard for 10/100TX protocol IEEE 802.3af Power over Ethernet PD |

## END OF SECTION