DIVISION 27 21 29

IFS – IEEE 802.3AT HIGH POWER OVER GIGABIT ETHERNET SPLITTER

POE302-SP

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- IFS POE302-SP IEEE 802.3at High Power Over Gigabit Ethernet Splitter

1.02 SECTION INCLUDES

- IFS POE302-SP IEEE 802.3at High Power Over Gigabit Ethernet Splitter – Standalone

1.03 REFERENCES

- Federal Communications Commission (FCC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provides 1 10/100/1000Base-T copper RJ-45 input port with IEEE 802.3at High Power over Ethernet and splits Ethernet data and DC power to network device that is not compliant with IEEE 802.3at standard.

- The system shall utilize EIA568, category 5/5e/6, 4-pair cables for 10Base-T, 100Base-TX or 1000Base-T to transmit Ethernet data and power simultaneously into the PoE Splitter.

- The module shall provide 12V or 24VDC power output based on the setting selected with built-in DIP switch.

1.05 SUBMITTALS

- Manufacturer’s Installation and Operating Manual: Printed installation and operating information for the PoE Splitter.

- Warranty: Manufacturer’s Printed Warranty.

1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment.

- Storage Temperature not to exceed: -20˚C to +70˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Product shall operate in an environment with an ambient temperature range of 0 Degree C to +50 Degree C without the assistance of fan-forced cooling.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 5% to 90% (non-condensing).

1.08 WARRANTY

- UTCFS Return and Warranty Policy: UTCFS warrants the product to be free of factory defects for period of 3 years.

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer:

- IFS Brand
UTC Fire & Security, Inc.
8985 Town Center Parkway
Bradenton, FL 34202-5129

- Phone 1-855-286-8889

- Email: presales@interlogix.com

- Substitutions: Not Permitted

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

- Model Compatibility Chart: Reference Table B: Product Compatibility Chart

2.03 GENERAL SPECIFICATIONS

- The Power over Ethernet Splitter shall be a POE302-SP module.

- The module shall comply with IEEE 802.3at Power over Ethernet standard.

- The module shall support the Ethernet data IEEE 802.3/802.3u/802.3ab 10/100/1000Base-T protocol using Auto-negotiating and Auto-MDI/MDI-X features.

- The module features two fixed 10/100/1000T electrical ports in a compact mini box.

- The port printed with “PoE In” functions as "PoE (Data and Power) Input".

- The port on the other side and printed with “Ethernet” functions as "Data output".

- The module features a fixed DC power jack and two selectable DC plug cables.

- The module shall provide adjustable two different output voltage options: 12V and 24V.

- The module shall receive power from either data (UTP PIN 1,2,3,6) or spare pairs (UTP PIN 4,5,7,8) by IEEE 802.3at standard

- The module shall support total distance up to 100 meters.

- The module shall provide power (PoE ready) status indicating LED’s for monitoring of system operation.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3

- Data Rate: 10/100 /1000 Mbps

- Data Inputs: 2

2.05 STATUS INDICATORS

A. LED Definition:

| LED | Color | Function |
| --- | --- | --- |
| PoE In | Green | Lights to indicate the port is receiving 56V DC in-line power. |

2.06 CONNECTORS

- Data:

- Ethernet Port: RJ-45

- Ethernet + DC Port: RJ-45

- Power: DC jack with 2.5mm receptacle in the central post

2.07 ELECTRICAL SPECIFICATIONS

- The PoE Splitter:

- Input Voltage: 52 to 56VDC

- DC output power:

- Output Voltage: 12VDC @ 2A or 24V@1A

2.08 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions (WxDxH): 3.74” x 2.75” x 0.98” (95 x 70 x 25mm)

- Finish: Module shall be constructed of a Metal enclosure.

- Weight < 0.24lbs / 111g

2.9 ENVIRONMENTAL SPECIFICATIONS

- MTBF: >50,000 Hours

- Operating Temp: 0˚ C to +50˚ C

- Storage Temp: -20˚ C to +70˚ C

- Relative Humidity: 5% to 90% (non-condensing).

2.10 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Federal Communications Commission (FCC) Part 15, Class A

- European Union Compliance (CE) with following standard:

- EN 55022:2006, Class B

- EN61000-3-2:2006

- EN61000-3-3+A2:2005

- EN 55024+A2:2003

2.11 ACCESSORIES

- 15cm UTP straight network cable x1

- DC Plug cable x 2

PART 3 - EXECUTION

3.01 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module. The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the installation manual.

3.02 INSTALLATION

- General: Locate PoE Splitter module as indicated on the approved detail drawings and install module in compliance with the IFS User’s manual.

- Connect a standard network cable from IEEE 802.3at complied PSE device to "PoE In" port of IFS POE302-SP. The POE In LED of POE302-SP will start to blink continuously.

- Connect the UTP cable in the package from "Ethernet" of IFS POE302-SP to the RJ-45 port of remote device.

- Connect proper DC plug from "DC OUT" of IFS POE302-SP to remote device.

- Power on the remote device. LED indicator on IFS POE302-SP will illuminate.

3.03 TESTING

- Testing the 10/100/1000T Gigabit Ethernet Copper Link.

- Verify that the data leads and UTP ports are properly connected.

- Make sure that power is applied to the PoE Splitter module or other equipment used in the system.

- Successful data link operation should be confirmed at this point by the communication established with connected equipment.

- Testing the 12V or 24V DC output capability.

3.05 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all UTP cable scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| IFS PART NO. | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| POE302-SP | IEEE 802.3AT HIGH POWER OVER GIGABIT ETHERNET SPLITTER | 300 feet (100M) electrical |

* Maximum distance is limited to optical loss of the fiber and any additional loss by connectors, splices and patch panels.

Table B: Product Compatibility Chart

| IFS PART NO. | COMPATIBLE ETHERNET EQUIPMENT |
| --- | --- |
| POE302-SP | IEEE 802.3/802.3u/802.3ab Ethernet standard for 10/100/1000T protocol IEEE 802.3at High Power over Ethernet PSE |

## END OF SECTION