DIVISION 27 21 29

IFS 16-Port 10/100BASE-TX 802.3at PoE + 2-Port Gigabit TP/SFP combo

Web Smart Switch

ES2402-16P-2C

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- IFS Sixteen Fast Ethernet PoE Plus Port and Two ports Gigabit TP/SFP combo Web Smart Switch

1.02 SECTION INCLUDES

- ES2402-16P-2C 16-port 10/100Mbps 802.3at PoE with 2 Shared SFP slot Web Smart Switch

1.03 REFERENCES

- Federal Communications Commission (FCC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provide 16 10/100BASE-TX copper ports with IEEE 802.3at Power over Ethernet Injector

- The system shall utilize EIA568, category 5/5e/6, 4-pair cables for 10BASE-T or 100BASE-TX to transfer Ethernet data and 52V DC power simultaneously.

- The system shall utilize 850nm to 1550nm optics capable of data transmission of 1000Mbps on multimode / single mode optical fibers.

- The Gigabit SFP ports can be optical 1000BASE-SX / LX through SFP (Small Form-Factor Pluggable) interface.

- The SFP module shall utilize 850nm optics capable of bi-directional data transmission of 1000BASE-SX on two multimode optical fibers.

- The SFP module shall utilize 1310nm optics capable of bi-directional data transmission of 1000BASE-LX on two single-mode optical fibers.

- The SFP module shall utilize 1310nm/1490nm or 1310nm/1550nm optics capable of bi-directional data transmission of 1000BASE-BX on one single-mode optical fiber.

- The ES2402-16P-2C Web Smart PoE Switch provides 16 10/100Mbps Fast Ethernet ports and 2 1000Mbps SFP interfaces with 16 IEEE 802.3at / 802.3af PoE injector.

- The PoE in-line power following the standard IEEE 802.3at / 802.3af makes the ES2402-16P-2C able to power on 16 PoE compliant devices at the distance up to 100 meters through the 4-pair Cat 5/5e UTP wire.

1.05 SUBMITTALS

- Manufacturer’s Installation and Operating Manual: Printed installation and operating information for the Web Smart PoE Switch.

- Warranty: Manufacturer’s Printed Warranty.

1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment.

- Storage Temperature not to exceed: -10˚ C to +70˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Products shall operate in an environment with an ambient temperature range of 0˚ C to +50˚ C with the assistance of fan-forced cooling.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 5% to 95% (non-condensing).

1.08 WARRANTY

- Standard UTC Fire & Security Inc. Comprehensive Warranty: UTC Fire & Security warrants the product to be free of factory defects under manufacture’s 3 Years Warranty.

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer:

- IFS Brand

UTC Fire & Security, Inc.

3211 Progress Drive,

Lincolnton, NC 28092

- Phone 1-855-286-8889

- Email: presales@interlogix.com

- Substitutions: Not Permitted

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

2.03 GENERAL SPECIFICATIONS

- The Web Smart PoE Switch shall be an ES2402-16P-2C model.

- The switch features 16 fixed 10/100TX electrical ports

- The switch features two 1000SX/LX optical SFP slots.

- The switch shall support the Ethernet data IEEE 802.3 protocol using Auto-negotiating and Auto-MDI/MDI-X features.

- The switch shall provide power, fan failure, link / act status and PoE In-use status indicating LED’s for monitoring proper system operation.

- The switch shall be a 1U (one U, 19 inches) 19-inch equipment.

- The switch shall be connected with EIA568A/B Cat 5/5e/6 UTP/STP cable system for its RJ45 interface ports.

- The ES2402-16P-2C shall comply with IEEE 802.3at / 802.3af Power over Ethernet:

- The ES2402-16P-2C shall support IEEE 802.3at Power over Ethernet detection and 52VDC power injection at port#1 to port#16

- The ES2402-16P-2C is also the power injectors which transmit DC Voltage to the Cat5/5e/6 cable and transfer data and power simultaneously to remote PD (Powered Device) equipments.

- The ES2402-16P-2C shall Auto-detect of PoE IEEE802.3at / 802.3af equipment; protect devices from being damaged by incorrect installation.

- The ES2402-16P-2C shall support total distance up to 100 meters on PoE ports.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3/3u/3ab/3z

- Data Rate:

- Port-1 to Port-16: 10/100Mbps

- Port-17 to Port-18 SFP: 1000 Mbps

- Data Inputs: 18

- Operation Mode: Simplex or Duplex

2.05 STATUS INDICATORS

- System

| 1. | PWR | Green | The switch unit is power on |
| --- | --- | --- | --- |

- 10/100BASE-TX Interfaces

| 1. | LNK/ ACT | Blink | Lit: indicate the link through that port is successfully established with speed 10Mbps or 100Mbps. |
| --- | --- | --- | --- |
|  |  |  | Off: indicate that the interface failed connecting to the network. |
|  |  |  | Blinking: The port is transmitting or receiving packets from the TX device. |
| 2. | PoE In-Use | Amber | Lit: The port is providing 52VDC in-line power |
|  |  |  | Off: The port connected device is not a PoE Powered Device (PD) or PoE inject capability be disabled by manual setting. |

- 1000T Copper Interfaces (Port-17 and Port-18)

| 1. | LNK/ ACT | Green | Lit: indicate the link through that port is successfully established at 1000Mbps. Blink: indicate that the switch is actively sending or receiving data over that port. |
| --- | --- | --- | --- |
| 2. | 1000 | Green | Lit: indicate that the port is successfully established at 1000Mbps. |
|  |  |  | Blink: indicate that the port is successfully established at 100Mbps. |
|  |  |  | Off: indicate that the port successfully established at 10Mbps. |

- 1000X SFP Interfaces (Shared with Port-17 and Port-18)

| 1. | LNK/ ACT | Green | Lit: indicate the link through that port is successfully established at 1000Mbps. Blink: indicate that the switch is actively sending or receiving data over that port. |
| --- | --- | --- | --- |
| 2. | 1000 | Green | Lit: indicate that the port is successfully established at 1000Mbps. |

- PoE Usage

| 1. | 50W | Amber | Indicates the PoE power consumption has equal 50W or over 50W. |
| --- | --- | --- | --- |
| 2. | 100W | Amber | Indicates the PoE power consumption has equal 100W or over 100W. |
| 3. | 150W | Amber | Indicates the PoE power consumption has equal 150W or over 150W. |
| 4. | 190W | Amber | Indicates the PoE power consumption has equal 190W or over 190W. |

2.06 CONNECTORS

- Optical: SFP slot

- Power: Universal AC socket

- Data: RJ45

2.07 ELECTRICAL SPECIFICATIONS

- Power Characteristics of ES2402-16P-2C:

- Voltage Input:100~240V AC / 50-60Hz

- Current: 4A max.

- Power Consumption: Maximum 220Watts with PoE full load

- PoE Output Power of ES2402-16P-2C:

- PoE output budget: 220Watts

- IEEE 802.3af class 3 (15.4W): Max. 14 ports

- IEEE 802.3at class 4 (30W): Max. 7 ports

2.08 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions: 17” x 7.8” x 1.75” (440mm x 200mm x 44.5mm)

- Finish: Module shall be constructed of a metal enclosure with a powder coat.

- Weight: 5.61bs./ 2.55kg

2.09 ENVIRONMENTAL SPECIFICATIONS

- MTBF: > 50,000 Hours

- Operating Temp: 0˚ C to +50˚ C

- Storage Temp: -10˚ C to +70˚ C

- Relative Humidity: 5% to 95% (non-condensing).

2.10 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Federal Communications Commission (FCC) Part 15, Class A

- European Union Compliance (CE) with following standard:

- EN 55022:2006, Class A

- EN61000-3-2:2006

- EN61000-3-3+A2:2005

- EN 55024+A2:2003

2.11 ACCESSORIES

- AC Power cord

- Rubber feet

- Rack-mount brackets

- SFP dusty caps

PART 3 - EXECUTION

3.01 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module.

- The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the installation manual.

- Rack Mount Module (19” Rack)

- The unit is installed in a standard EIA 19” (482.6 mm) rack or wall standoff bracket adequate for the size and weight of the rack mount unit. The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the user’s manual.

- Optical Fibers

- Caution: NEVER look into the end of an active optical fiber when using laser light output. Eye damage can occur. Wear eye protection when cleaving, terminating, and splicing fiber.

- The number of optical fiber SFP slot shall meet the requirements of the UTC Fire & Security model number.

- All optical fiber cables shall be properly installed and terminated with the mating optical connectors.

- The optical link shall be tested with either a power meter, at a minimum, or OTDR to ensure the link budget (overall path loss) plus an added 3dB of optical safety margin does not exceed the optical power budge.

3.02 INSTALLATION

- General: Locate fiber optic modules as indicated on the approved detail drawings and install module in compliance with the UTC Fire & Security User’s manual.

3.03 TESTING

- Testing the 10/100TX Fast Ethernet Copper Link.

- Verify that the data leads and UTP ports are properly connected.

- Make sure that power is applied to the PoE switch.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

- Testing the 10/100TX PoE Copper output capability.

3.04 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all UTP cable scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| IFS PART NO. | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| ES2402-16P-2C | 16-Port Fast Ethernet PoE+ plus 2 RJ45/SFP Combo Ports Web Smart Managed Ethernet Switch | 300 feet (100M) electrical |

Table B: Product Compatibility Chart

| SFP Transceiver | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| MULTI-MODE |  |  |
| S30-2MLC | SFP-Port 1000BASE-SX Mini-GBIC Module - 2 Fiber - 550m - Multi-Mode - 850nm (0~50℃) - Based on 50/125µm OM2 Fiber | 550m |
| S30-2MLC-2 | SFP-Port 1000BASE-SX2 Mini-GBIC Module - 2 Fiber - 2Km - Multi-Mode - 1310nm (0~50℃) - Based on 50/125µm OM4 Laser Optimise | 2Km |
| SINGLE MODE |  |  |
| S30-2SLC-10 | SFP-Port 1000BASE-LX10 Mini-GBIC Module - 2 Fiber - 10Km - Single-Mode - 1310nm (0~50℃ | 10Km |
| S30-2SLC-30 | SFP-Port 1000BASE-LHX Mini-GBIC Module - 2 Fiber - 30Km - Single-Mode - 1310nm (0~50℃) | 30Km |
| S30-2SLC-70 | SFP-Port 1000BASE-ZX Mini-GBIC Module - 2 Fiber - 70Km - Single-Mode - 1550nm (0~50℃) | 70Km |
| S30-1SLC/A-10 | SFP-Port 1000BASE-BX10 Mini-GBIC Module - 1 Fiber - 10Km - Single-Mode - Tx 1310nm - Rx 1490nm (0~50℃) | 10Km |
| S30-1SLC/B-10 | SFP-Port 1000BASE-BX10 Mini-GBIC Module - 1 Fiber - 10Km - Single-Mode - Tx 1490nm - Rx 1310nm(0~50 ℃) | 10Km |
| S30-1SLC/A-20 | SFP-Port 1000BASE-BX20 Mini-GBIC Module - 1 Fiber - 20Km - Single-Mode - Tx 1310nm - Rx 1490nm (0~50℃) | 20Km |
| S30-1SLC/B-20 | SFP-Port 1000BASE-BX20 Mini-GBIC Module - 1 Fiber - 20Km - Single-Mode - Tx 1490nm - Rx 1310nm (0~50℃) | 20Km |
| S30-1SLC/A-60 | SFP-Port 1000BASE-BX60 Mini-GBIC Module - 1 Fiber - 60Km - Single-Mode - Tx 1310nm - Rx 1490nm (0~50℃) | 60Km |
| S30-1SLC/B-60 | SFP-Port 1000BASE-BX60 Mini-GBIC Module - 1 Fiber - 60Km - Single-Mode - Tx 1490nm - Rx 1310nm (0~50℃) | 60Km |

* Maximum distance is limited to optical loss of the fiber and any additional loss by connectors, splices and patch panels.

## END OF SECTION