DIVISION 27 21 29

IP55 802.11b/g/n 2.4GHz 300Mbps Outdoor WLAN AP/Bridge/Router Built in 12dBi

WMC251-1W-1T-300

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- IP55 802.11b/g/n 2.4GHz 300Mbps Outdoor WLAN AP/Bridge/Router Built in 12dBi

1.02 SECTION INCLUDES

- WMC251-1W-1T-300 IP55 802.11b/g/n 2.4GHz 300Mbps Outdoor WLAN AP/Bridge/Router Built in 12dBi

1.03 REFERENCES

- Federal Communications Commission (FCC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provide 2 10/100BASE-TX RJ45 ports, and with one port Proprietary Power over Ethernet capability.

- The system shall utilize EIA568, category 5/5e, 4-pair cables for 10Base-T or 100Base-TX to transfer Ethernet data and 24V DC power simultaneously.

- The RF module can be 2.4GHz 2T2R through wireless interface.

- The RF module shall utilize 802.11b/g/n 2.412GHz to 2.484GHz frequency band capable of data transmission maximum PHY data rate up to 150Mbps using 20MHz bandwidth, 300Mbps using 40MHz.

- The RF module shall utilize the built-in 12dBi dual-polarization antenna capable of 2*2 MIMO data transmitting/receiving on 30 degrees Horizontal Beamwidth and 20 degrees Vertical Beamwidth of wireless interface.

- The RF module shall utilize OFDM with BPSK, QPSK, 16-QAM, 64-QAM modulation types.

- The WMC251-1W-1T-300 Outdoor WLAN CPE provides 2 10/100Mbps Fast Ethernet ports with IEEE 802.11b/g/n wireless interface.

- The LAN 1 port supports Proprietary Power Over Ethernet makes the WMC251-1W-1T-300 able to be powered by its Proprietary POE injector at the distance up to 30 meters through the 4-pair Cat 5/5e UTP wire.

1.05 SUBMITTALS

- Manufacturer’s Installation and Operating Manual: Printed installation and operating information for the Outdoor WLAN CPE.

- Warranty: Manufacturer’s Printed Warranty.

1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment.

- Storage Temperature not to exceed: -30˚ C to +80˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Products shall operate in an environment with an ambient temperature range of -20˚ C to +70˚ C.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 10% to 95% (non-condensing).

1.08 WARRANTY

- Standard UTC Fire & Security Inc. Comprehensive Warranty: UTC Fire & Security warrants the product to be free of factory defects under manufacture’s 3 Years Warranty.

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer:

- IFS Brand

UTC Fire & Security, Inc.

8985 Town Center Parkway

Bradenton, FL 34202-5129

- Phone 1-855-286-8889

- Email: presales@interlogix.com

- Substitutions: Not Permitted

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

2.03 GENERAL SPECIFICATIONS

- The Outdoor WLAN CPE shall be an WMC251-1W-1T-300 model.

- The CPE features 2 fixed 10/100TX RJ45 ports

- The CPE features 802.11b/g/n 2.4GHz 2*2 MIMO 300Mbps WLAN interface.

- The CPE shall support the Ethernet data IEEE 802.3 protocol using Auto-negotiating and Auto-MDI/MDI-X features.

- The CPE shall provide Power, LAN 1, LAN 2 and Wireless status indicating LED’s for monitoring proper system operation.

- The CPE shall be IP55 UV-resistance outdoor equipment.

- The CPE shall be connected with EIA568A/B Cat 5/5e UTP/STP cable system for its RJ45 interface ports.

- The WMC251-1W-1T-300 shall comply with IEEE 802.11b/g/n wireless technology:

- The WMC251-1W-1T-300 shall supports IEEE 802.11n 2*2 MIMO capable of transmitting/receiving maximum PHY 300Mbps data rates.

- The WMC251-1W-1T-300 shall backward compatible IEEE 802.11b/g standards capable of comply to 802.11b/g clients.

- The WMC251-1W-1T-300 shall supports

Proprietary Power over Ethernet detection

and 24VDC power injection at LAN port#1.

- The WMC251-1W-1T-300 shall support total distance up to 30 meters on PoE LAN ports (LAN 1).

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3/3u/3x

- Data Rate:

- RJ45 Port-1 to Port-2: 10/100Mbps

- WLAN: 300Mbps

- Operation Mode: Simplex or Duplex

2.05 STATUS INDICATORS

- System

| 1. | PWR | Green | The CPE unit is power on |
| --- | --- | --- | --- |

- 10/100BASE-TX Interfaces

| 1. | LAN1 LAN2 | Green | Lit: indicate the link through that port is successfully established with speed 10Mbps or 100Mbps. |
| --- | --- | --- | --- |
|  |  |  | Off: indicate that the interface failed connecting to the network. |

- Wireless Interfaces

| 1. | Wireless | Green | Lit: indicate that the interface is successfully initialized. |
| --- | --- | --- | --- |
|  |  |  | Off: indicate that the interface is failed to initialized or disabled. |

2.06 CONNECTORS/BUTTONS

- Grounding Terminal

- Data + Power:  RJ45 (LAN1)

- PIN assignment: PIN 4, 5 (+ Vdc); PIN 7, 8 (- Vdc); PIN 3 (Reset)

- Data: RJ45 (LAN2)

- Reset

2.07 ELECTRICAL SPECIFICATIONS

- Power Characteristics of POE Injector:

- Voltage Input:100~240V AC / 50-60Hz

- Current: 0.6A max.

- PoE Output Power of POE Injector:

- PoE output:24V DC, 1A

- PIN assignment: PIN 4, 5 (+ Vdc); PIN 7, 8 (- Vdc); PIN 3 (Reset)

2.08 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions: 5” x 10” x 2.5” (127mm x 254mm x 63mm)

- Finish: Module shall be constructed of IP55 UV-resistance enclosure.

- Weight:1.03lb / 465g

2.09 ENVIRONMENTAL SPECIFICATIONS

- MTBF: > 50,000 Hours @25˚ C

- Operating Temp: -20˚ C to +70˚ C

- Storage Temp: -30˚ C to +80˚ C

- Relative Humidity: 10% to 95% (non-condensing).

2.10 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Federal Communications Commission (FCC) Part 15, Class B

- European Union Compliance (CE) with following standard:

- EN 301 489-1 V1.9.2

- EN 301 489-17 V2.2.1

- EN 300 328 V1.8.1

- EN 60950-1

2.11 ACCESSORIES

- 24V POE injector

- AC Power cord

- Protective Plug

- Plastic Strap

PART 3 - EXECUTION

3.01 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module.

- The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the installation manual.

- Grounding

- Do not install the device close to any electrical grounding device or lightning protection system. Place the device’s own grounding and lightning protection system apart from any electrical grounding device and lightning protection system as far as possible.

- Keep the device far from high-power radio transmitter, radar transmitter, high-frequency, and high-current devices.

- Make sure to provide an excellent grounding system to guarantee the stable operation of device, as well as to protect it from lightning, interference and electrostatic discharges.

- POE Injector

- Caution: Must disconnect the surge arrestor inline before resetting the CPE through the reset button on the POE Injector.

- Supply stable power to the device. Unstable power may cause the device to malfunction.

3.02 INSTALLATION

- General: Locate grounding wiring in compliance with the UTC Fire & Security User’s manual.

3.03 TESTING

- Testing the 10/100TX Fast Ethernet Copper Link.

- Verify that the data leads and UTP ports are properly connected.

- Make sure that power is applied to its Proprietary PoE injector.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

- Testing the 2.4GHz Wireless Broadcasting capability.

- Testing the 2.4GHz Wireless Performances.

3.04 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all UTP cable scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| IFS PART NO. | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| WMC251-1W-1T-300 | IP55 802.11b/g/n 2.4GHz 300Mbps Outdoor WLAN AP/Bridge/Router Built in 12dBi | 100 feet (30M) electrical |

* Maximum distance is limited to optical loss of the fiber and any additional loss by connectors, splices and patch panels.

## END OF SECTION