DIVISION 27 21 29

1200Mbps 802.11ac Dual Band Ceiling Mount Wireless Access Point

WMC303-1W-1T-1200

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

## 1.01 SUMMARY

- 1200Mbps 802.11ac Dual Band Ceiling Mount Wireless Access Point

## 1.02 SECTION INCLUDES

- WMC303-1W-1T-1200 1200Mbps 802.11ac Dual Band Ceiling Mount Wireless Access Point

## 1.03 REFERENCES

- Federal Communications Commission (FCC)

- European Union Compliance (CE)

## 1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provide one 10/100/1000BASE-T RJ45 port with Power over Ethernet capability.

- The system shall utilize EIA568, category 5/5e/6, 4-pair cables for 10BASE-T or 100BASE-TX and 1000BASE-T to transfer Ethernet data and 48~56V DC power simultaneously.

- The RF module can be 2.4GHz 300Mbps and 5GHz 867Mbps through wireless interface simultaneously.

- The 2.4GHz RF module shall utilize 802.11b/g/n 2.412GHz to 2.484GHz frequency band capable of data transmission maximum PHY data rate up to 150Mbps using 20MHz bandwidth, 300Mbps using 40MHz.

- The 5GHz RF module shall utilize 802.11a/n/ac 5.180GHz to 5.850GHz frequency band capable of data transmission maximum PHY data rate up to 173.3Mbps using 20MHz bandwidth, 400Mbps using 40MHz, 867Mbps using 80MHz.

- The RF module shall utilize the built-in two 2.5dBi 2.4GHz PCB antennas and two 4dBi 5GHz PCB antennas capable of 2*2 MIMO data transmitting/receiving of wireless interface.

- The RF module shall utilize OFDM with BPSK, QPSK, 16-QAM, 64-QAM, 256-QAM modulation types.

- The WMC303-1W-1T-1200 11ac Dual Band Ceiling Mount Wireless Access Point provides one 10/100/1000Mbps Gigabit Ethernet PoE/PD port with IEEE 802.11a/b/g/n/ac wireless interface.

- The LAN port supports Power Over Ethernet makes the WMC303-1W-1T-1200 able to be powered by IEEE 802.3af/at POE switch/injector at the distance up to 100 meters through the 4-pair Cat 5/5e/6 UTP wire.

## 1.05 SUBMITTALS

- Manufacturer’s Installation and Operating Manual: Printed installation and operating information for the 11ac Dual Band Ceiling Mount Wireless Access Point.

- Warranty: Manufacturer’s Printed Warranty.

## 1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment.

- Storage Temperature not to exceed: 0˚ C to 50˚ C

## 1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Products shall operate in an environment with an ambient temperature range of 0˚ C to 50˚ C.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 10% to 90% (non-condensing).

## 1.08 WARRANTY

- Standard UTC Fire & Security Inc. Comprehensive Warranty: UTC Fire & Security warrants the product to be free of factory defects under manufacture’s 3 Years Warranty.

# PART 2 - PRODUCTS

## 2.01 MANUFACTURER

- Acceptable Manufacturer:

- IFS Brand

UTC Fire & Security, Inc.

3211 Progress Drive,

Lincolnton, NC 28092

- Phone 1-855-286-8889

- Email: presales@interlogix.com

- Substitutions: Not Permitted

## 2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

## 2.03 GENERAL SPECIFICATIONS

- The Wireless Access Point shall be a WMC303-1W-1T-1200 model.

- The AP features 1 fixed 10/100/1000T RJ45 port

- The AP features 802.11a/b/g/n/ac 2.4GHz 2*2 MIMO 300Mbps and 5GHz 2*2 MIMO 867Mbps WLAN interface.

- The CPE shall support the Ethernet data IEEE 802.3 protocol using Auto-negotiating and Auto-MDI/MDI-X features.

- The CPE shall provide Power/ Position bi-color indicating LED’s for monitoring proper system operation.

- The CPE shall be connected with EIA568A/B Cat 5/5e/6 UTP cable system for its RJ45 interface ports.

- The WMC303-1W-1T-1200 shall comply with IEEE 802.11a/b/g/n/ac wireless technology:

- The WMC303-1W-1T-1200 shall supports IEEE 802.11n 2*2 MIMO capable of transmitting/receiving maximum PHY 300Mbps data rates in 2.4GHz and 867Mbps data rates in 5GHz.

- The WMC303-1W-1T-1200 shall backward compatible IEEE 802.11a/b/g standards capable of comply with 802.11a/b/g clients.

- The WMC303-1W-1T-1200 shall supports

IEEE 802.3at/af Power over Ethernet and

capable be powered by PSE (Powered

Source Equipment).

- The WMC303-1W-1T-1200 shall support total distance up to 100 meters on PoE (LAN) port.

## 2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3/3u/3ab/3x

- Data Rate:

- RJ45 Port: 10/100/1000Mbps

- WLAN: 300Mbps (2.4GHz); 867Mbps (5GHz)

- Operation Mode: Simplex or Duplex

## 2.05 STATUS INDICATORS

- System

| 1. | PWR | Green | On: the device power on |
| --- | --- | --- | --- |
|  |  |  | Off: the device power off (control by S/W) |
|  |  | Orange | On: system is initializing, turned it off when system boot up completed |
| 2. | Position | Orange | On: locate the device’s position and the LED blinks continuously. |
|  |  |  | Off: the LED is off. |

## 2.06 CONNECTORS/BUTTONS

- Data: RJ45

- Reset button

## 2.07 ELECTRICAL SPECIFICATIONS

- Power Characteristics of WMC303-1W-1T-1200:

- Voltage Input: 802.3af/at PoE 48V DC

- Current: 0.5A max.

## 2.08 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions (Φ x H): 7.6” x 1.9” (194mm x 49mm)

- Finish: Plastic

- Weight:0.6lb / 280g

## 2.09 ENVIRONMENTAL SPECIFICATIONS

- MTBF: > 50,000 Hours @25˚ C

- Operating Temp: 0˚ C to 50˚ C

- Storage Temp: -40˚ C to +70˚ C

- Relative Humidity: 10% to 90% (non-condensing).

## 2.10 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Federal Communications Commission (FCC) Part 15, Class B

- European Union Compliance (CE) with following standard:

- EN 301 489-1 V1.9.2

- EN 301 489-17 V2.2.1

- EN 300 328 V1.8.1

- EN 301 893 V1.7.1

- EN 60950-1

## 2.11 ACCESSORIES

- Mounting Bracket

- Mounting Kit

# PART 3 - EXECUTION

## 3.01 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module.

- The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the installation manual.

- POE

- Supply stable power to the device. Unstable power may cause the device to malfunction.

## 3.02 INSTALLATION

- General: General: mount the device in compliance with the UTC Fire & Security User’s manual.

## 3.03 TESTING

- Testing the 10/100/1000T Gigabit Ethernet Copper Link.

- Verify that the data leads and UTP port is properly connected.

- Make sure that PoE power is applied to the

WMC303-1W-1T-1200.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

- Testing the 2.4GHz and 5GHz Wireless Broadcasting capability.

- Testing the 2.4GHz and 5GHz  Wireless Performances.

## 3.04 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all UTP cable scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| IFS PART NO. | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| WMC303-1W-1T-1200 | 1200Mbps 802.11ac Dual Band Ceiling Mount Wireless Access Point | 300 feet (100M) electrical |

* Maximum distance is limited to 100 meters.

END OF SECTION