DIVISION 27 21 29

IFS 8-Port Gigabit Ethernet Industrial Unmanaged Switch

NS3050-8T

ENGINEERING SPECIFICATIONS

# PART 1 - GENERAL

1.01 SUMMARY

- IFS Eight Gigabit Ethernet Port Industrial Unmanaged Switch

1.02 SECTION INCLUDES

- NS3050-8T 8-Port Gigabit Industrial Unmanaged Switch

1.03 REFERENCES

- Federal Communications Commission (FCC)

- European Union Compliance (CE)

1.04 SYSTEM DESCRIPTION

- Performance Requirements: Provide 8 10/100/1000BASE-T copper Ethernet ports

- The system shall utilize EIA568, category 5/5e/6, 4-pair cables for 10BASE-T, 100BASE-TX and 1000BASE-T to transfer Ethernet data.

1.05 SUBMITTALS

- Manufacturer’s Installation Manual: Printed installation information for the Gigabit Ethernet Switch.

- Warranty: Manufacturer’s Printed Warranty.

1.06 DELIVERY, STORAGE AND HANDLING

- Store in original packaging in a climate controlled environment.

- Storage Temperature not to exceed: -40˚ C to +75˚ C

1.07 PROJECT/SITE CONDITIONS

- Temperature Requirements: Products shall operate in an environment with an ambient temperature range of -40˚ C to +75˚ C with the assistance of fan-forced cooling.

- Humidity Requirements: Products shall operate in an environment with relative humidity of 5% to 95% (non-condensing).

1.08 WARRANTY

- Standard UTC Fire & Security Inc. Comprehensive Warranty: UTC Fire & Security warrants the product to be free of factory defects under manufacture’s 3 Years Warranty.

# PART 2 - PRODUCTS

2.01 MANUFACTURER

- Acceptable Manufacturer:

- IFS Brand

UTC Fire & Security, Inc.

8985 Town Center Parkway

Bradenton, FL 34202-5129

- Phone 1-855-286-8889

- Email: presales@interlogix.com

- Substitutions: Not Permitted

2.02 MANUFACTURED UNITS

- Model Number Descriptions: Reference Table A: Product Number Descriptions

2.03 GENERAL SPECIFICATIONS

- The Web Smart PoE Switch shall be an NS3050-8T model.

- The switch features 8 fixed 10/100/1000T electrical ports

- The switch shall support the Ethernet data IEEE 802.3 protocol using Auto-negotiating and Auto-MDI/MDI-X features.

- The switch shall be connected with EIA568A/B Cat 5/5e/6 UTP/STP cable system for its RJ45 interface ports.

2.04 DATA SPECIFICATIONS

- Data Interface: Ethernet IEEE802.3/3u/3ab

- Data Rate:

Port-1 to Port-8: 10/100/1000Mbps

- Data Inputs: 8

- Operation Mode: Simplex or Duplex

2.05 STATUS INDICATORS

- System

| 1. | P1, P2 | Green | The switch unit is power on |
| --- | --- | --- | --- |

- 10/100/1000BASE-T Interfaces

| 1. | LNK/ ACT | Blink | Lit: indicate the link through that port is successfully established with speed 10Mbps, 100Mbps or 1Gbps. |
| --- | --- | --- | --- |
|  |  |  | Off: indicate that the interface failed connecting to the network. |
|  |  |  | Blinking: The port is transmitting or receiving packets from the TX device. |
| 2. | 1000 | Green | Lit: The port is established with speed 1000Mbps |
|  |  |  | Off: established with speed 10Mbps or 100Mbps. |

2.06 CONNECTORS

- Power:  Universal AC socket

- Data: RJ45

2.07 ELECTRICAL SPECIFICATIONS

- Power Characteristics of NS3050-8T:

- Voltage Input:12 to 48VDC

- Current: 1.2A max.

- Power Consumption: Maximum 6.72 watts with full load

2.08 MECHANICAL SPECIFICATIONS

- Surface Mount Dimensions: 5.3” x 3.4” x 1.3” (135mm x 87mm x 32mm  W x D x H)

- Finish: Module shall be constructed of a metal enclosure with a powder coat.

- Weight: 1.01bs./ 0.46kg

2.09 ENVIRONMENTAL SPECIFICATIONS

- MTBF: > 100,000 Hours

- Operating Temp: -40˚ C to +75˚ C

- Storage Temp: -40˚ C to +75˚ C

- Relative Humidity: 5% to 95% (non-condensing).

2.10 REGULATORY AGENCIES/APPROVALS AND LISTINGS

- Federal Communications Commission (FCC) Part 15, Class A

- European Union Compliance (CE) with following standard:

- EN 55022:2006, Class A

- EN61000-3-2:2006

- EN61000-3-3+A2:2005

- EN 55024+A2:2003

2.11 ACCESSORIES

- Wall-mount brackets

- RJ45 dusty caps

PART 3 - EXECUTION

3.01 PREPARATION

- Standalone Module (Surface Mount)

- Shall be mounted on a properly prepared surface adequate for the size and weight of module.

- The placement of the unit shall allow provision for cable installation and maintenance as indicated on the approved detail drawings and in compliance with the installation manual.

3.02 INSTALLATION

- General: Locate wall-mount modules as indicated on the approved detail drawings with the UTC Fire & Security User’s manual.

3.03 TESTING

- Testing the 10/100/1000T Gigabit Ethernet Copper Link.

- Verify that the data leads and UTP ports are properly connected.

- Successful data link operation should be confirmed at this point by communicating with other equipment.

3.04 CLEANING

- Follow all instructions for proper use of solvents and adhesives used for termination and splicing.

- At completion of the installation, dispose of all UTP cable scraps properly.

# MANUFACTURED UNITS REFERENCE TABLES

Table A: Product Number Descriptions

| IFS PART NO. | DESCRIPTION | MAX. DISTANCE* |
| --- | --- | --- |
| NS3050-8T | 8-Port Gigabit Ethernet Industrial Unmanaged Switch | 300 feet (100M) electrical |

## END OF SECTION