# Saving Video from Digia to USB Pen drive

- Press Play / Pause

- Set the Year, Month, Day and Hour (24 hour mode) and press “SEL” button

- Select the Channel or ALL channels. Press “SEL”

- Select Type or ALL types. Press “SEL”

- Select the result of interest. Press “SEL”

- Play video till start of event. Press Pause to pause video

- Press USB / Archive button. Select Still or Video. Press “SEL”

- Set amount of seconds you want to save. Press “SEL”

**NOTE THE CURRENT TIME**

*The video has now been marked but not saved yet.*

- Press “ESC” four times to show Live video

- Press “FF” button

- Set the Year, Month, Day and Hour of when you marked the video.  (note:  If the hour is left at 00, it will show all marked video for the entire day)

- Press “SEL”

- Highlight the result from the list and press “SEL”. You will view the first frame of video.

- Insert USB Pen Drive

- Press USB / Archive button. Choose YES and press “SEL”. This will save video / still to the pen drive.

- Press “ESC” four times to back out to live video

- Remove pen drive when complete.